Thomas Greve

Clemens August Stra�e 53

D 5300 Bonn 1

Germany

eMail:	greve@dbnpib5.bitnet

Here you have what you wanted to have ever since you bought

your TSENG-chip based VGA card: a .BGI driver for its 256

color modes! This driver supports the following modes:

Mode	Resolution	Video-Mem required

0	 320x200	 256K

1	 640x480	 512K

2	 800x600	 512K

If you want to use it, make sure, you really own a VGA-card

with a TSENG chip ET3000 on it and it is really equipped

with 512K (16 chips 41464). If there are 8 of these chips

and 8 empty sockets, go buying another 8. They're about

$2.50 each ($20 total) these days.

The driver itself doesn't test if it can handle the VGA

card in your computer. So if you are not sure, remove the

cover of your computer... (see your VGA manual for details

on how to remove the cover.)

The following cards are equipped with the ET3000 chip set:

EIZO MDB 10

GENOA 5x00 / 6x00 (?)

Orchid Designer

You may look at your vpic.cfg file. If your chip set is

marked as TSENG there and vpic works with this configuration

in 800*600*256 mode, you most probably own an ET3000 based

VGA card.

The following cards are *definitely not* equipped with the

ET3000 chip set:

Any cards from Ahead, ATI, Chips & Technologies, Everex,

OAK, Paradise, Trident, Video 7 and True Blue VGAs.

This driver will work on '286+ machines only. Even on a

12MHz AT it will be a bit slow - there are MBytes of data

to be moved and the ET3000 allows a maximum data transfer

rate of 250 KByte/s. (It is not the driver nor the computer,

that limits speed!)

DISCLAIMER:

Of course, I do not take any warranty whatsoever for this

driver, neither expressed nor implied etc, etc... Especially

I don't take any warranty that it works with your VGA, that

it doesn't screw up your monitor or that your VGA must be

equipped with 41464 RAMs.

The above names of VGA cards are trademarks of their

respective manufacturers. vpic is copyrighted by Bob

Montgomery.

You may use this driver for your personal purposes only.

If you copy it for your friends, upload it to bulletin

boards etc., please include this readme-file. I won't give

any technical support. If you want to sell this driver with

your application programs, please contact me.

ANNOUNCEMENT:

If you own a '386 computer, you may get a '386-Version of

this driver: it supports 32 bit line patterns (useful in

800x600+ modes!) and uses some '386 codes.

If you have a ET4000 based VGA card, this driver won't run

on it properly. You may get an ET4K Version of this driver

with an additional 1024x768 mode ('286 or '386 version, see

above) as well. 1 MB video mem is required for this mode.

If you want one of the above drivers, send me a US-$10

cheque (US-$15 outside Europe, 15DM inside Germany) and i'll

send you the driver. Please note the chipset (ET3K/ET4K),

the CPU type and the disk format (5 1/4" or 3 1/2").

For $20 ($25 outside Europe) you can get all of the three

drivers.

INSTALLATION:

Due to a bug in the GRAPH unit of TP 5.X / the

graphics.lib of TC 2.00 this driver has to be installed as

follows:

var

Driver, Mode : integer;

{ ... }

Driver := InstallUserDriver('ET3000', nil) + 5;       { the 5 is necessary }

Mode := Mode_Of_Your_Choice;				{ due to the bug... }

InitGraph(driver, Mode, GraphPath);

if GraphResult < 0 then begin

Writeln('ERROR: ET3000 driver not installed!');

Halt(1);

end;

{ ... }

For installation with TC++ 1.01 / TP 6.00 see readmeXX.XXX.

NOTES: - GetMaxColor returns 255, but SetColor(GetMaxColor);

may yield in unpredictible results, since the

colors 248..255 are not initialized by default.

Initialize these colors with SetRgbPalette or

use SetColor(White).

- GetModeRange doesn't work for third party drivers.

You've got to use GetMaxMode instead.

- FloodFill doesn't work with 256 color modes (as it

doesn't with the IBM8514 driver supplied by

BORLAND) with TC 2.0 / TP 5.X. Don't ask me why. It

works with the TP 6.00 / TC++ 1.01, though.

- XORput isn't implemented except for PutImage.

- CharSize of DefaultFont is ** 8x16 **. The driver

gets this font from your VGA-BIOS. If garbage

appears on the screen, it's a bug of your VGA-BIOS.

If no text appears at all or if it doesn't fit, you

have assumed TextHeight('') to return 8.

Proper programming required. ;-)

- Any suggestions about improvements are welcome. Bug

reports are useless since there aren't any bugs.

(hm? Did i hear you laughing?? ;-)

OK. There *were* some bugs in version 1.00, but

none of you found out...

- ...

CREDITS: Terje Thoegersen has tested this driver thoroughly,

i thank him for his never-ending patience when

waiting for the next release.