PrintGL  1.28

pen plotter emulator for IBM/DOS systems

(C) Copyright Ravitz Software Inc. 1990,1992

Ravitz Software Inc.                              BBS/FAX 606-268-0577

P.O. Box 25068

Lexington, KY  40524-5068

USA                                  Compuserve Cary Ravitz [70431,32]

Introduction ....................................................... 2

License and Registration                                             3

What Is New ........................................................ 4

Installation                                                         4

Operation .......................................................... 5

How To Get HP-GL Plotfiles From Your Graphics Software               6

Messages ........................................................... 6

Syntax                                                               7

Overview of Options ................................................ 8

/A - Location Option                                                 8

/B - Tiling Option ................................................. 8

/C - Pen Color Option                                                9

/D - Output Destination Option .................................... 10

/F - Output Format Option                                           11

/I - Scaling Point Location Option ................................ 13

/J, /K - Printer Code Prefix, Suffix Options                        13

/L - Page Layout Option ........................................... 14

/M - Magnification Option                                           15

/N - Page Number Option ........................................... 15

/O - Origin and Orientation Option                                  15

/P - Plotfile Option .............................................. 16

/Q - Number of Copies Option                                        16

/R - Internal Resolution Option ................................... 16

/S - Pen Shading Option                                             16

/T - Temporary File Option ........................................ 16

/W - Pen Width Option                                               17

/X - User Interface Option ........................................ 17

/Y - D/Y and S/E Switch Settings Option                             17

/Z - Chord Angle and Enhanced Font Option ......................... 18

PrintGL Menu Interface (PMI)                                        19

PMI Specific Functions ............................................ 19

PMI Input and Output Choices                                        20

PMI Plot Options .................................................. 20

PMI Pen Options                                                     20

PMI Page Layout Options ........................................... 21

PMI Other Options                                                   21

Large Paper ....................................................... 21

Tips On AutoCAD                                                     22

Answers ........................................................... 22

Useful Prefix and Suffix Codes                                      23

Color Processing .................................................. 24

HP-GL Commands                                                      24

Output Devices .................................................... 25

Compatibility and Technical Information                             32

User Support ...................................................... 32

p-2

Introduction ---------------------------------------------------------

PrintGL is a shareware pen plotter emulator for IBM/DOS systems.  It

prints or displays HP-GL (Hewlett-Packard Graphics Language -  7475,

17440  subsets) plotfiles  on most  PC graphics  devices.  HP-GL  is

widely  supported  by CAD,  drawing, plotting,  and  other  graphics

programs  including  AutoCAD,  Microstation,  Generic CADD, MathCAD,

Schema, OrCAD, and  SAS.  PrintGL will print on Epson  and IBM 9 pin

printers, Epson, Fujitsu,  IBM, NEC, and Toshiba 24 pin printers, HP

LaserJet,   DeskJet,  color  DeskJet,  PaintJet,  QuietJet,  HP-GL/2

devices, and HP-GL  plotters,  IBM  ExecJet,  Quietwriter  2 and  3,

LaserPrinter,  and  plotters, Canon LBP, BJ, and BJC printers, Kodak

Color  4,  PDP   ProTracer,  Tektronix  ColorQuick,  and  PostScript

printers.  It will also display plots with a CGA, EGA, VGA, enhanced

VGA, or HGC,  and output a ZSoft PCX bit map file or an Encapsulated

PostScript file with preview bit map.

Even  if  your  graphics program supports  your  printer,  you  will

probably  find that PrintGL  is faster,  gives better print quality,

and/or  gives more  formatting flexibility.  PrintGL  uses  the best

graphics modes available for each printer that  it supports and uses

transfer  data compression  to improve print speed on many printers.

And it gives options  such as orientation, magnification,  position,

and pen  width, color, and  shading.  If  you need  printed graphics

output from personal software, you can output  HP-GL and use PrintGL

to do the printing.  This gives immediate support to a wide range of

printers.  And it lets you use the HP 17440 command  set while still

supporting lower function plotters.

PrintGL interprets all of  the HP 7475 and 7440/17440 (ColorPro with

GEC)  command  set  that  is  applicable  to  plotfiles  except  for

character set 8 (Katakana).  None of the digitize, output, or device

control  commands (except plotter on and off) are supported as these

are only used  with direct control of a plotter.  And the error mask

command is not supported.  In addition to the HP 7475 and 7440/17440

commands, PrintGL handles paper feed, label origin, extra space, and

enhanced clipping window commands and proportional fonts.

PrintGL can  be  run  from the DOS  command line (and .BAT files) or

from PrintGL Menu Interface (PMI). PMI is a menu driven program that

lets  you  choose  options,  select  a  list  of plotfiles,  and run

PrintGL.  This  simplifies the selection of options and lets you run

multiple plotfiles  without  intervention.  PMI  lets  you save five

configurations,  so after initial setup you can use it with  minimal

effort.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to  make  sure  that  the  shareware

principle  works   for  you.   If  you   are  unable  to  resolve  a

shareware-related  problem  with  an ASP  member  by  contacting the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an  ASP member, but does not

provide technical support for members' products. Please write to the

ASP  Ombudsman at  545  Grover Road,  Muskegon, MI 49442  or send  a

CompuServe message via CompuServe Mail to ASP Ombudsman 70007,3536.

p-3

License and Registration ---------------------------------------------

PrintGL (including PrintGL Menu Interface) is provided as is.  There

are no warranties expressed or implied.

PrintGL is copyrighted by Ravitz Software Inc.  You may copy PrintGL

only for use under this license agreement.

PrintGL  is  distributed  as shareware.  You may use PrintGL without

charge on a trial basis to determine its suitability for you. If you

continue to  use it  after  your  evaluation, you  must  purchase  a

registered copy for $50.  This  registration covers  use by a single

person (on multiple computers) or installation  on a single computer

(may be used by a group of people).  There is no cost to use a later

1.** version after registering any 1.** version.

You may distribute PrintGL  as a stand alone product if you keep the

entire   package   together,  unchanged,   including   this  license

explanation, and do not charge more than $10. You may not distribute

PrintGL as part of another product.

To  purchase a registered copy of PrintGL,  send your name, address,

and $50 check to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

You  will be sent  a printed  manual  and  disk (always  the  latest

version).  Specify  3.5  or  5.25  inch  disk.   If  you  previously

registered  PrintGL, you may send  $15  to  get a current  disk  and

manual.

The price for multiple licenses is $50 for the first copy (with disk

and  manual) and thereafter $40 per copy with disk and manual or $25

per copy without disk or manual.

For users outside the USA, please make checks payable in US dollars,

and except for  Canada,  please  use  a  check  from  a  US  bank if

possible. Don't forget to include your country in your address.

If you register through a business, be sure to include your name.

PrintGL Printer Driver is a special version of PrintGL, modified for

use as a  printer driver.  It may be licensed for  use in commercial

programs. Send a note to request more information on this.

p-4

What Is New ----------------------------------------------------------

For version 1.28:

multiple file output (see /D option)

supports the LaserJet 4 at 600 dpi

supports the DeskJet 550C and PaintJet XL300

pen widths can be up to 24 dots

Ctrl-B (instead of ctrl-break) stops processing immediately

For version 1.26:

GEM IMG output (/FG) has been dropped

low resolution (180 dpi) Canon BJ, BJC drivers have been dropped

/FG drives the Pacific Data Products ProTracer

/FU is an improved HP LaserJet 2 driver

/F7+ is a two pass (double ink) BJ/ExecJet driver

/FB+ is a BJC driver with sparse line to line overlap

/FV* drives VESA EVGA 1024x768 displays

/FV+ (VESA EVGA 800x600) is fixed

/F0 drives any HP-GL/2 raster device

/F8 adds supports for the HP 7220

the PostScript XP macro (set via prefix code) sets the paper tray

the nominal resolution of PS, HP-GL, HP-GL/2 may be specified

line width N turns a pen off

the O suboption of /L has been dropped

pen overwrite mode (opaque pens) is the default

The latest version of PrintGL is available from the support bulletin

board at  606-268-0577  (1200..9600,N,8,1  24 hours/day,  XMODEM  or

YMODEM (1K XMODEM) download), packed in .ZIP format.

Installation ---------------------------------------------------------

Copy  the files PRINTGL.EXE and PMI.EXE  to  your  working  disk  or

directory. They are all that you need to run PrintGL.

PRINTGL.EXE is the command line  driven executable file.  PMI.EXE is

the executable file for PrintGL Menu Interface. PMI needs to be able

to find  itself (PMI.EXE)  and PRINTGL.EXE,  so  both of these files

should be in a directory that is in your  DOS PATH.  If  you prefer,

you can set the environment  variable  PRINTGLP  to be a  path  that

tells PMI where to find itself and PRINTGL.EXE. If you have PRINTGLP

defined, PMI ignores the PATH.

In addition to the above files, these files are part in the package,

and must be included if you distribute PrintGL as shareware:

PRINTGL.DOC  documentation

ADD.DOC      additional documentation since last release

HPGL.DOC     HP-GL reference

HPGL.CHR     HP-GL character set plotfile

SAMPLE.PLT   sample plotfile

ORDER.FRM    order form

README.*     quick information

p-5

Operation ------------------------------------------------------------

PrintGL reads a plotfile, does arc,  character, fill, etc.  to  line

conversion,  stores the data in  a  coordinate list, and  if needed,

puts the list on disk. It then reorients the data and determines the

plot size. Next the data is resized and relocated. For vector output

formats (HP-GL, PostScript) data is output from here. For bit mapped

devices  (most printers, displays, bit maps) the coordinate list  is

rasterized and output in swaths.  You are likely to  see the printer

print a swath and stop while PrintGL is processing the next swath.

Esc  stops PrintGL at the next good break point, leaving the printer

ready for new data. Ctrl-B stops PrintGL as quickly as possible. You

generally need to do a printer reset after a Ctrl-B.

When you  display a  plot, press Enter or  Esc  to return to the DOS

text screen,  or  any  of  the four  cursor movement  keys to scroll

across the plot.  Press S to cycle between full, half (the default),

quarter, and eighth page scrolling.

PrintGL works within a printer's current margins  and starts at  the

current  vertical print  position.  If  you decline  the  form  feed

suboption  of the  /L option then PrintGL leaves the  printer at the

start of the  line directly below the plot.  All printing is done in

printer lines that cover the full height of the printhead.

PrintGL will not automatically change a printer's operating mode. On

multimode  printers, the  right  operating  mode must be  set before

running PrintGL or with the prefix code (/J) option.

Internally PrintGL  uses two byte  coordinates  on  a 1016 dots/inch

grid. This allows for plots with coordinates of over 30 inches - far

larger than an HP 7475 can handle. However, if you are using PrintGL

to emulate a large plotter, you may overflow the internal coordinate

system.  This condition  is  not checked, and  it will cause garbage

output.  You can change the internal resolution to 508 dots per inch

(giving 60 inch maximum plot sizes) with the /R option (/R508). This

only helps  store the  original plot  internally.  The  next step in

PrintGL converts  these  coordinates  to the  requested size  in the

resolution of the output device, and these coordinates must also fit

in the two byte coordinate list.

p-6

How To Get HP-GL Plotfiles From Your Graphics Software ---------------

To  use PrintGL, you must  be  able to output HP-GL files from  your

graphics application.  Most CAD, drawing, and graphing software lets

you do this.  Just tell the application to output your  picture to a

plotter.  In either the configuration process or the output process,

you will need to specify that you  have an HP 7475  or HP 7440/17440

plotter  and that you want the output to go to a  file  instead of a

COM port (some programs name the file automatically and some ask you

for a name).  Once you have the file, you can print it with PrintGL,

either from a DOS shell or after exiting the application.

Messages -------------------------------------------------------------

PrintGL  displays a number of messages to  keep you informed of what

it is doing. The output looks like this.

Reading SAMPLE.PLT

Plotfile has 1 page(s)    Processing page 1    2283 data records

Overall magnification    0.9964

Print window   Horizontal   0.00   7.54    Vertical   0.00  10.14

Plot area      Horizontal   0.42   7.13    Vertical   0.61   9.54

Output format /FT  HxV dpi nominal 180x180, actual 180x180  color

Writing to device LPT3#

The first  line  is the plotfile  that is currently being processed.

The second line tells how many pages are in the plotfile, which page

is being processed, and the number of data points in that page.  The

number of  data points  is roughly  equivalent  to the number of pen

moves and  is  shown  to  give  you  an  indication  of  the  plot's

complexity.

The  overall magnification is  either  the  magnification  that  you

specified  with the /M option or the  effective  magnification after

fitting to the print  window with /MF.  The  print window line gives

the horizontal  and vertical coordinates of the print window, always

starting  at 0,0 and in inches.  The plot area  gives the horizontal

and vertical coordinates of a box that just covers the plotted data.

This uses the same coordinate system as the print window, so you can

tell where the plot is relative to the print window.

The output format and  resolution are  given  and  then  the  output

destination is noted  (except for screen  plots).  If three  or four

plane color  is being used, the word  "color" is  displayed.  If you

have a black only printer, but the output format allows color  (this

is  common  with 9 and  24 pin  printers),  using color will greatly

increase the PrintGL run time with no improvement in print quality.

During  processing, PrintGL informs  you when  it is  processing and

writing data,  and you can watch the progress from 0% to 100%, along

with the elapsed time.

p-7

Syntax ---------------------------------------------------------------

The calling syntax of PrintGL is:

PRINTGL plotfile /option1 /option2 ...

Spaces between parameters  are  not required.  The  plotfile  may be

specified with  the /P option if it  is inconvenient to make it  the

first parameter.

If  a given option is entered more than once, only the last entry is

used.  And if  an  option  is  invalid  (but  not incorrect), it  is

ignored.  This lets you set  up a  batch  file with  your  preferred

options but override them as desired.  For  example, create PLOT.BAT

with the line:

PRINTGL /FL /M1 /P %1 %2 %3 %4 %5 %6 %7 %8 %9

Then  PLOT can be called  just  like PRINTGL but it  defaults to  HP

LaserJet format with magnification 1.

The DOS environment variable PRINTGL  is examined by PrintGL and  if

it  is  nonblank  then  it  is  put  in  front  of  the command line

parameters.  If you  put "SET PRINTGL=/FL/M1"  in your  AUTOEXEC.BAT

then  PrintGL will default to LaserJet format with  magnification 1.

This is ignored when running under PrintGL Menu Interface.

If you do not enter any parameters or if any parameter is incorrect,

PrintGL displays its option reminder screen.

Below, [ and ] enclose optional suboptions  and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except  that multiple numeric suboptions  are  ordered  and  numeric

suboptions must follow character suboptions where the characters may

be digits.  "c"  refers to  a  suboption character,  "x"  and "y" to

suboption  numbers,  "i"  and "j"  to  suboption  integers, "b" to a

suboption byte, and "f" to a DOS file or device name.

For  numbers (x, y), a decimal point is allowed but is not required,

and scientific notation is not allowed.  "-" is allowed but not "+".

For  integer input (i, j), only  base  ten integers  from  -32768 to

32767 are allowed. "+" is not allowed. A byte (b) must be a base ten

integer  from  0 to 255 or a hexadecimal number prefixed with $ from

$00 to $FF.  Further range checking is done for most options to flag

unreasonable  input.  If an option  has multiple numeric suboptions,

these may be separated with commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

p-8

Overview of Options --------------------------------------------------

PrintGL's options fall into several categories.  The  first is input

and output choices.  The  input is chosen with /P, the plotfile, and

/N,  the  page number.  Output options  are  /F,  the output  format

(printer model), and /D, the destination port, device, or file.

The  second category is the  page layout.  /L controls  the size and

location of the  print window (hard clip  limits), and the option to

box  the window, send  a form feed, reverse the image color, and use

opaque lines. Tiling is set with /B and copies with /Q.

The third category  defines  how  the  plot  looks  within the print

window.  /M,  the  magnification, sets the  size  of  the  plot.  /A

determines the location of the plot within the print window. /O sets

the  origin and orientation, and  is  used to  rotate  or mirror the

plot. /I sets the plotter's initial scaling points.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

And  the last category determines  how  PrintGL processes  the plot.

This includes the  temporary  file /T, the default and minimum chord

angle /Z, the  plotter Y/D and S/E switch position  /Y, the internal

resolution  /R, user interface options /X, and /J and /K let you set

up and reset the printer for special situations.

/A - Location Option -------------------------------------------------

/A[O][x,y] - location - default /A

The A option determines the location of the plot in the print window

via  a point on  the plot that  is aligned with a point in the print

window.  You can specify the plot alignment point with x,y in inches

from the plot's  origin  (before  PrintGL applies magnification), or

let  it default to the plot  center.  For the print window alignment

point, you can default to the center or specify the origin with O.

/A      puts the center of the plot at the center of the print

/AO0,0  puts the plot 0,0 at the print window origin

/B - Tiling Option ---------------------------------------------------

/B[i,j,x] - tiling parameters - default /B1,1,0

/B lets  you specify tiling  options that break  a large  plot  into

pages.  Each page is defined by the page layout option (/L). I and j

specify the number of tiles  (pages) horizontally and vertically.  X

specifies the overlap in inches.  Displayed plots are sized based on

the  tiling, but  only the first tile is displayed.  Use  the cursor

keys to scroll the plot.

/B2,2,.5  breaks the plot into 4 pages with .5 inch overlap

/B        is the same as /B1,1,0 and produces no tiling

p-9

/C - Pen Color Option ------------------------------------------------

/C[O|T]c.. - color - default /CK

/C specifies the color of  each of 40  pens.  The colors  are  B for

blue, C for cyan, G  for green, K for  black,  M for magenta, R  for

red, W for  white, and Y for yellow.  Unspecified pens use the  last

selected color (/CRGB  is  the same as /CRGBBBBBB).  For black/white

output formats, all nonwhite colors become black.

Additional colors can be created with shade mixing. These colors use

shading patterns  to  mix  the  eight pure colors.  The format for a

mixed color is  .??  where each ?  represents any of  the eight pure

colors.  The first  color is printed  with the pen's shading pattern

(/S option)  and the  second  is printed  with  the  inverse shading

pattern.  If  the shade value is  0 (solid) it is replaced by 2 (one

dot checkerboard) which produces the most uniform mixed colors.

Mixed  colors  need line widths  of at least 2 (4 for  2 dot shading

patterns) to  look good, and they may have visual artifacts on edges

near a 45 degree angle.  Multipass dot matrix formats may not handle

color mixing well.

By default, colors  are overwritten  into the print, so they  appear

opaque.  You can  switch to translucent colors (ored into the print)

with the T suboption, and then back to opaque with the  O suboption.

Make  this mode switch in front  of the pen that you want to change.

For PostScript devices and plotters, the color processing is done by

the device, not PrintGL.  PostScript devices use  opaque  colors and

plotters use translucent colors.

" means repeat the previous color.

/COKR.MB""T.KY  sets pen 1     to opaque black

pen 2     to opaque red

pen 3..5  to opaque magenta on blue

pen 6..40 to translucent black on yellow

p-10

/D - Output Destination Option ---------------------------------------

/D[+]f - destination - default /D1

The D option specifies the output printer port, device, or file. The

+ suboption causes files to be  appended rather than overwritten (it

has no effect on device output). 1, 2, and 3 select the BIOS printer

routines.  X1, X2, X3, and X4 select xon/xoff controlled serial port

drive.  H1,  H2, H3, and H4 select  hardware  controlled serial port

drive.  You  can also  use the  DOS devices LPT1, COM1,  etc.  /D is

ignored for display output.

PrintGL cannot set up the serial ports.  This is usually done in the

AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

If you specify a three digit number for  a file name extension  (for

example .000), and multiple pages are output via the multiple copies

option, tiling, or multiple HP-GL pages, then  rather than appending

the destination file, additional files  are created  with sequential

extensions. This is useful for PCX and EPS output.

/D2           sends output to the second parallel port via BIOS

/DLPT3        sends output to the DOS device LPT3

/D+TEMP.PRN   appends output to file TEMP.PRN

/DOUTPUT.000  sends output to OUTPUT.000, OUTPUT.001, ...

p-11

/F - Output Format Option --------------------------------------------

/Fc[-|+|*][!][~][`][x,y] - output format - default /FN~

The F option specifies the output format. -, +, and * are resolution

modifiers, !  selects a compression mode driver,  ~  turns off color

processing, and ` switches four  plane  color processing on  or off.

These suboptions are allowed even if they have no effect.

For 800x600 VGA (/FV+) you can append the display mode number.  This

must follow the format character.  It may be entered in hex with a $

prefix. For HP-GL (/F8), HP-GL/2 (/F0), and PostScript (/FS) you can

append  the  nominal   device  resolution.   Matching   the  nominal

resolution to the printer may yield improvements in print quality.

You  can append the horizontal and vertical resolution of the output

device  in  dots/inch.  This  overrides  the  normal  output  device

resolution, but does not affect the structure of  the output.  These

numbers  should follow all other suboptions.  In addition to letting

you  choose any  resolution  for a bit  map,  this  feature lets you

compensate for print size error.  For example, a 360x360 dpi printer

consistently under  indexes so that a 6 inch high box comes out 5.94

inches.  Using 360,363.6 (6/5.94*360 = 363.6) removes the error. For

print  size correction, you also specify a number from .5 to 2 which

is interpreted  as  the nominal  resolution times  this number.  For

example on a 360 dpi printer, 1.0,1.01 is the same as 360,363.6.

Drivers  marked  below  with  !  have an optional  compression  mode

driver, selected with "!". This may be useful but it depends on your

printer.  These drivers compress the output data by breaking bit map

strips  with  printhead  positioning  commands.   This   saves  data

transmission time, but some printers spend more time positioning the

printhead than is saved by not sending the data.

A  number of  printers use  compression  mode by default.  It can be

turned off with � (ASCII 173).

Drivers  marked below with  ~  can handle color.  Using ~ suppresses

color processing, substituting black for all colors except white.  `

switches between 4 and 3 plane color (see Color Processing).

/F0     ~ HP-GL/2 ......................................... 600x600

/F0i    ~ HP-GL/2                                             ixi

/F1     ~ IBM 9 pin ....................................... 120x72

/F1+    ~ IBM 9 pin (2 pass)                                120x144

/F1*    ~ IBM 9 pin (4 pass) .............................. 240x144

/F2       IBM Quietwriter 2                                 240x240

/F3       IBM Quietwriter 3 ............................... 240x240

/F4-      IBM LaserPrinter (PPDS mode)                      150x150

/F4       IBM LaserPrinter (PPDS mode) .................... 300x300

/F5    !~ IBM 24 pin alternate 1                            180x180

/F5+   !~ IBM 24 pin alternate 1 (2 pass) ................. 360x180

/F5*   !~ IBM 24 pin alternate 1 (4 pass)                   360x360

/F6    !  IBM Proprinter X24 .............................. 180x182

/F6+   !  IBM Proprinter X24 (2 pass)                       360x182

/F6*   !  IBM Proprinter X24 (4 pass) ..................... 360x364

/F7       Canon BJ (Proprinter mode), IBM ExecJet           360x360

p-12

/F7+      Canon BJ (Proprinter mode), IBM ExecJet (2 pass)  360x360

/F8       HP-GL                                            1016x1016

/F8i      HP-GL enhanced (7550 enhanced mode) ............... ixi

/F9    !~ NEC 24 pin                                        180x180

/F9+   !~ NEC 24 pin (2 pass) ............................. 360x180

/F9*   !~ NEC 24 pin (4 pass)                               360x360

/FA-   !~ HP DeskJet 500C, Mannesmann Tally MT92C ......... 150x150

/FA    !~ HP DeskJet 500C, Mannesmann Tally MT92C           300x300

/FB     ~ Canon BJC 800 ................................... 360x360

/FB+    ~ Canon BJC 800 (line overlap)                      360x360

/FC       CGA display .....................................  62x25

/FD-      Canon LBP (ISO mode)                              150x150

/FD       Canon LBP (ISO mode) ............................ 300x300

/FE     ~ 128K EGA color display                             62x45

/FF    !~ Fujitsu 24 pin .................................. 180x180

/FF+   !~ Fujitsu 24 pin (2 pass)                           360x180

/FF*   !~ Fujitsu 24 pin (4 pass) ......................... 360x360

/FG       PDP Protracer (Proprinter mode)                   360x360

/FH       Hercules graphics card display ................... 70x45

/FI-      HP LaserJet 3, DeskJet                            150x150

/FI       HP LaserJet 3, DeskJet .......................... 300x300

/FI+      HP LaserJet 4                                     600x600

/FJ       Canon BJ (BJ130 mode) ........................... 360x360

/FK     ~ Kodak Diconix Color 4                             192x192

/FL-      HP LaserJet ..................................... 150x150

/FL       HP LaserJet                                       300x300

/FM       128K EGA monochrome display .....................  62x45

/FN     ~ Epson 9 pin                                       120x72

/FN+    ~ Epson 9 pin (3 pass) ............................ 120x216

/FN*    ~ Epson 9 pin (6 pass)                              240x216

/FO    !~ Toshiba 24 pin .................................. 180x180

/FO+   !~ Toshiba 24 pin (2 pass)                           360x180

/FO*   !~ Toshiba 24 pin (4 pass) ......................... 360x360

/FP    !~ HP PaintJet                                       180x180

/FQ       HP QuietJet (PCL mode) .......................... 192x192

/FR     ~ Tektronix ColorQuick                              216x216

/FS     ~ PostScript ...................................... 600x600

/FSi    ~ PostScript                                          ixi

/FT    !~ Epson 24 pin .................................... 180x180

/FT+   !~ Epson 24 pin (2 pass)                             360x180

/FT*   !~ Epson 24 pin (4 pass) ........................... 360x360

/FU-      HP LaserJet 2                                     150x150

/FU       HP LaserJet 2 ................................... 300x300

/FV     ~ VGA display                                        62x62

/FV+    ~ VESA 800x600 VGA display ......................... 78x78

/FV+b   ~ 800x600 VGA display - mode b                       78x78

/FV*    ~ VESA 1024x768 VGA display ....................... 100x100

/FW     ~ Epson 9 pin alternate                             120x72

/FW+    ~ Epson 9 pin alternate (3 pass) .................. 120x216

/FW*    ~ Epson 9 pin alternate (6 pass)                    240x216

/FX    !~ IBM 24 pin alternate 2 .......................... 180x180

/FX+   !~ IBM 24 pin alternate 2 (2 pass)                   360x180

/FX*   !~ IBM 24 pin alternate 2 (4 pass) ................. 360x360

/FY       append Encapsulated PostScript preview bit map     72x72

/FYi,j    append Encapsulated PostScript preview bit map .... ixj

/FZ     ~ ZSoft PCX                                         100x100

/FZi,j  ~ ZSoft PCX ......................................... ixj

p-13

/I - Scaling Point Location Option -----------------------------------

/IA|R|W[N] - IP point location - default /IA

This option is not useful with any HP-GL that does not use the scale

command or that initializes the plotter or sets the scaling points.

The  I option  sets the initial scaling points (P1, P2 - the scaling

points  may  be set  in  the HP-GL  file  with  the IP  command).  A

(absolute) sets the points  to the default  for  the paper size (see

below).  R fits P1 and P2 to the print window with at least .25 inch

margins  and an x/y ratio of 10/7.2 (the same ratio as  the HP  7475

with small paper).  W fits P1 and P2  to the print window with a .25

inch margin. N makes the margin 0 for R or W and is ignored with A.

The  default scaling points depend on the print window  (/L  option)

with the implied paper size and the plot orientation (/O option).

paper    options            default scaling pts  with RO90 command

7440 A   /O1 /L 7.54,10.14  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O1 /L 7.54,10.74  250,279 10250, 7479  279,250  7479,10250

7475 A4  /O1 /L 7.84,10.20  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O1 /L 7.60,10.88  603,521 10603, 7721    0,610  7200,10610

7475 B   /O1 /L10.20,16.38  522,259 15722,10259  283,934 10283,16134

7475 A3  /O1 /L10.88,15.90  170,602 15370,10602  607,797 10607,15997

7440 A   /O2 /L10.14, 7.54  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O2 /L10.74, 7.54  250,279 10250, 7479  279,250  7479,10250

7475 A   /O2 /L10.20, 7.84  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O2 /L10.88, 7.60  603,521 10603, 7721    0,610  7200,10610

7475 B   /O2 /L16.38,10.20  522,259 15722,10259  283,934 10283,16134

7475 A3  /O2 /L15.90,10.88  170,602 15370,10602  607,797 10607,15997

anything else               250,279 10250, 7479  279,250  7479,10250

/O3 and /O4 gives the same default scaling points as /O1 and /O2.

/IA   uses the default scaling points

/IWN  sets the scaling points to fit the print window

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

/J lets you send codes to the printer before PrintGL's usual data is

sent and /K  lets you send codes after PrintGL's usual data is sent.

This lets you set up and reset the  printer for special  situations.

For  HP-GL  and   PostScript  output,   the  /J  codes   follow  the

initialization commands.

The  codes  are  specified  with  byte  values  (0..255  or $0..$FF)

separated with blanks or commas.  There is  a limit of 80 characters

for any option, so very long  sequences must  be handled outside  of

PrintGL.

/J27 79   sets skip perforation off  to allow plots  larger than one

page on most continuous form printers

p-14

/L - Page Layout Option ----------------------------------------------

/L[B][F][R][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the print window edge using pen 40.

F and N set form feed or no form feed.  Form feed is the default and

does not need  to be specified.  This is ignored for PCX and display

output.

R  reverses the colors on displays, bit maps, and output  formats 4,

D, I, L, and U (Canon, HP, and IBM laser printers) and is  otherwise

ignored. This gives the equivalent of a photographic negative.

The first optional x and y are the print window width and height  in

inches.  These  define the hard clip limits.  These  values are  not

checked against the printer's capabilities.  If you specify  a print

window  that  exceeds the  output  device  limits,  the  results are

unknown.  You may need to reset the printer's margins or page length

for large plots (see Large Paper). The size needed for a given width

and height exceeds x and y by 8 dots to allow for line widths.

The  second optional x and y are the left and  top margins, measured

from the  printer's left margin and current vertical position to the

print window. The margins for HP-GL and PostScript are measured from

0,0 (left, bottom).  Margins are ignored for  display output and bit

maps.

THE PRINT  WINDOW PLUS THE  MARGINS MUST  FIT  WITHIN THE  PRINTABLE

AREA.

Each printer has a default page layout.  The default print window is

7.54x10.14 for printers and bit maps  and  10.14x7.54 for  HP-GL and

displays.  These correspond to HP 7440 A size paper. The margins are

printer  specific  to compensate for different  paper  handling.  In

evaluating  user  options, PrintGL  processes each  option and  when

finished  reprocesses the default page layout for the chosen printer

and  then reprocesses the last user /L option.  This procedure gives

good  default values  so that your /L option can  change the  letter

suboptions while keeping the default print window.

/L7.5,3,.5,0   print window 7.5x3, left margin .5, top margin 0

/L7.54,10.14   sets window equivalent to HP 7440 with A  paper

/L7.54,10.74   sets window equivalent to HP 7440 with A4 paper

/L7.84,10.20   sets window equivalent to HP 7475 with A  paper

/L7.60,10.88   sets window equivalent to HP 7475 with A4 paper

/L10.20,16.38  sets window equivalent to HP 7475 with B  paper

/L10.88,15.90  sets window equivalent to HP 7475 with A3 paper

p-15

/M - Magnification Option --------------------------------------------

/M[F]x - magnification - default /MF.9

The M option specifies the magnification.  F fits the  plot  to  the

print window size.  The default is /MF.9 which makes the plot 90% of

the largest size that will  fit in the print  window.  /M1 gives the

same size plot as an HP 7475 plotter.

/MF.9  fits the plot to the print window and shrinks it 10%

/M1    plots at the same size as an HP 7475

/M.5   plots at half the size of an HP 7475

/N - Page Number Option ----------------------------------------------

/N[i] - page number - default /N

/Ni says to print only page i of the plotfile. If you do not specify

i then all pages in the plot will be printed (this is the default).

/N   prints all pages in the plotfile

/N3  prints only page 3

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O

The O  option  sets  the print  window origin  and orientation.  The

origin  is  one of four corners numbered 1..4  for upper left, lower

left, lower right, and upper right, and implies a plot rotation. The

L suboption specifies a left handed coordinate system, which mirrors

the plot.  If no origin is specified then it is chosen so that the x

axis is the longer dimension.

Here are pictures of PrintCAD's view of the output media compared to

an HP 7475's view.

HP 7475                               PrintGL

display or

A size           B size               printer       plotter

y��������Ŀ    0,0��������Ŀy       ul �����Ŀur   ul ��������Ŀur

�        �       �        �         1 �     �4     1 �        �4

�        �       �        �           �     �        �        �

0,0����������x      �        �           �     �     ll ����������lr

�        �           �     �      2           3

�        �        ll �������lr

�        �         2        3

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

p-16

/P - Plotfile Option -------------------------------------------------

/Pf - plotfile - no default

The plotfile may be specified as the first parameter with no prefix,

or  it may be specified anywhere in the option list with /P.  If you

use a  file name  mask with "*"  and/or  "?",  each matching file is

processed, but the first failure or user break stops all processing.

/Q - Number of Copies Option -----------------------------------------

/Qi - number of copies - default /Q1

/Q  sets the number of copies of  each printed page.  The default is

one.  Page printers  produce multiple  copies  faster using  the  "N

copies" prefix code (see Useful Prefix and Suffix Codes).

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R508.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

/S assigns a shading pattern  to each pen.  Unspecified pens use the

last specified shade.  The shade values  are 0..8.  0 is the default

solid line.  1..4 are 1 dot patterns (75%, 50%, 25%, 12.5%) and need

line  widths of at least three to give good looking lines.  5..8 are

2x2 dot patterns (75%,  50%,  25%, 12.5%) and need line widths of at

least five to give good looking lines. Some printers, especially dot

matrix printers  in  multipass modes,  may wash  out shading because

they have a dot size that is large relative to the dot spacing.

1     2     3     4       5         6         7         8

����  ����  ��       �    ������    ��  ��    ��              ��

����  ����    ��   �      ��  ����    ��  ��    ��

����  ����  ��       �    ����  ��  ��  ��          ��    ��

����  ����    ��   �        ������    ��  ��      ��

/S024  sets pen 1 to solid, pen 2 to 50%, pen 3..40 to 12.5%

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPLOT.TMP

PrintGL uses a temporary  file when there is  not  enough  memory to

handle a plotfile.  You can use /T to put it on a RAM disk for extra

speed.  You can  force the use  of a temporary file with + ( if  the

plot  fills the  buffer).  If you  use  a RAM disk  this  may  speed

printing.

p-17

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

The W option assigns pen line widths. Each width may be 0..9, .10 ..

.24 dots, A..I for 2..10 units of 4/1016 inch (.1  mm), or N to turn

off the  pen.  A..I widths are  approximations, based  on the device

technology. Odd widths of 9 or above are modified to the next higher

(even) width. Unassigned pens use the last assigned width.

For nonsquare matrix printers, PrintGL uses pen points optimized for

an h/v ratio of .67, 1, or 1.5.  Widths are figured in the direction

of higher resolution, and are not accurate in the other direction.

Line widths  of over eight dots are  distorted at  the print  window

edge to fit within the four dot border.

/WA      sets pens 1..40 to .2 mm

/WB2.16  sets pen 1 to .3 mm, pen 2 to 2 dots, pens 3..40 to 16 dots

/X - User Interface Option -------------------------------------------

/X[C][P][S][F][U] - interface options - default /X

The X option sets various interface options. C causes a break when a

plot is clipped. P causes a pause before each printed page.

S suppresses  location and  size messages.  F  suppresses  location,

size, file name, and page number messages.  U suppresses unsupported

command and incorrect number messages.

/XS  gives only the most important messages

/Y - D/Y and S/E Switch Settings Option ------------------------------

/Y[D|Y][S|E] - D/Y and S/E switch setting - default /YDS

The Y option sets the plotter's D/Y  switch to  D  or Y and the  S/E

switch to S or  E.  Y starts  with the  plotter off and responds  to

esc.(, esc.Y, esc.), and esc.Z commands.  D  starts with the plotter

on and ignores these commands. S sets the standard interpretation of

IW (clipping window) commands.  E sets 7550 enhanced  interpretation

of IW commands, so that they use scaled coordinates.

/YD  puts the YD switch in the usual D position

p-18

/Z - Chord Angle and Enhanced Font Option ----------------------------

/Z[i,j][E] - default, min chord angle, enhance font - default /Z10,5

This option is not useful with any  HP-GL that  draws circles, arcs,

and  text  as line segments rather than using HP-GL circle, arc, and

label commands.

The Z option specifies  the default  and  minimum chord  angles  for

circles and arcs. The HP 7475 defaults to a chord angle of 5 with no

minimum.  Using chord  angles  of less than  10 returns little print

quality improvement  and  adds to  processing time.  The  default is

/Z10,5. For better emulation use /Z5,2.

The E suboption says to  use  the enhanced  font, which doubles  the

number of chords on curves in text,  improving the curve smoothness.

This is only effective on character heights above .5 cm.

/Z5,2  sets the default chord angle to 5 degrees, the minimum to 2

p-19

PrintGL Menu Interface (PMI) -----------------------------------------

PrintGL Menu Interface  (PMI) is a menu driven program that lets you

select PrintGL options, choose a list of plotfiles, and run PrintGL.

To run  PrintGL Menu  Interface, enter  PMI at the  DOS prompt,  and

press Enter. There are no command line options. The PMI main menu is

displayed at the top of the screen, and the bottom  of the screen is

used  differently for each main menu  option.  The  PMI main menu is

divided by column into six groups. Each group is described below.

When the cursor is on a  main menu  item, the current  value  of the

option is displayed below the main menu.  To change the value, press

Enter (or left mouse  button).  The options are modified with cursor

movement and typing numbers/names.  The Enter key returns you to the

main menu with the new  option values.  Esc (or  right mouse button)

returns you to the main menu with the option values unchanged.

If  one of the first two letters of a main menu item is capitalized,

that item may be accessed immediately by pressing the letter.

When a cursor bar has a notch, the value in the notch may be edited.

PMI Specific Functions -----------------------------------------------

The first column of main menu options is dedicated to PMI functions.

These   include  running   PrintGL,  switching   to   an   alternate

configuration, exiting PMI,  saving  the current PMI configurations,

setting  PMI options, and creating a  BAT file corresponding to  the

current PMI configuration.

The  "Alt config" item rotates between five PMI configurations.  The

current configuration name is on line 2.

The "save configs" item saves all of the current PMI configurations.

The plotfile  list  is not saved.  PMI  saves its  configuration  by

writing  to  PMI.EXE.  To  find  PMI.EXE it  looks  in  the  current

directory  and  then  searches the  DOS  PATH,  saving to  the first

PMI.EXE that it finds.

The  "menu options" item lets you  set the configuration name, mouse

sensitivity,  screen colors, up  to five initial plotfile masks, and

the name of the BAT file created by the "create BAT" option.  Screen

colors are specified with the standard IBM BIOS attributes.

For color displays, the left hex  digit is the  background color and

the  right hex digit is the foreground color.  Bright colors are not

available in the background.

0 black       4 red           8 gray (bright)     C bright red

1 blue        5 magenta       9 bright blue       D bright magenta

2 green       6 brown         A bright green      E bright yellow

3 cyan        7 white         B bright cyan       F bright white

"Create BAT" creates a  BAT  file  that  can  be  run  with "batfile

hpglfile /opt1 /opt2 ..".  The BAT file contains the options defined

by the current PMI configuraton and accepts additional options.

p-20

PMI Input and Output Choices -----------------------------------------

Column  two of the main menu  lets  you choose the plotfiles to plot

(the /P option  in  PrintGL),  the plot page number (/N), the output

destination (/D),  the output format  (/F), and modifications to the

output - compression mode, device resolution override and prefix and

suffix printer codes (/J,/K).

You may enter up to 99 plotfiles in the "plotfile" main menu option.

To  choose from a  list of files, enter a  mask name and press Enter

(the cursor must  be on the  mask).  PMI will then display a list of

corresponding  files.  Select  any number of the files by moving the

cursor  to the desired file and pressing Space  (or the middle mouse

button). If no files have been selected, then pressing Enter selects

the file under the cursor.  The selected files are added to the file

list at  or below  the  cursor.  Files that would overflow the  99th

position on the plotfile list are dropped.

When you run PrintGL, PMI runs PrintGL for each file on the plotfile

list.  Masks are ignored.  For any  plotfiles that PrintGL does  not

successfully process, PMI inserts an arrow in front of the  plotfile

name.  You  can view the results from the "plotfile" main menu item.

The arrow is not considered  part of the file name by PMI, so you do

not need to remove it to retry PrintGL.

Printers that  support  color  have  a  color  option ("clr:" on the

menu).  This lets you specify a  black/white printer even though you

may have set  up colors with the  "pen color"  menu.  Output formats

that do not support color do not have this option and always convert

the selected colors to black and white.

PMI Plot Options -----------------------------------------------------

The third  column of  main  menu options  define  how the  plot will

appear on the  page.  The options include magnification (/M), origin

(/O), position (/A), and initial scaling points (/I).

Left handed (mirrored) orientations are not available from PMI.

PMI Pen Options ------------------------------------------------------

The  fourth   column   of   main   menu  options   covers   the  pen

characteristics: color (/C),  shade (/S),  and width  (/W).  Each of

these menus use the up and down cursor keys to select a pen, and the

left and right cursor keys to choose an attribute. Only fifteen pens

are supported in PMI.

For  color  selection,  the  one character  colors (b,c,g,k,m,r,w,y)

represent  pure  colors,  and  the  two character  colors are  shade

pattern mixes.  The  shade pattern  mixed  colors  use  the  current

shading  option  for  that  pen,  unless the  shade  pattern is 100%

(solid), in which case a one dot checkerboard is used.

p-21

PMI Page Layout Options ----------------------------------------------

The fifth column of main  menu options covers the  page layout.  The

first three items are suboptions of the /L option - print window and

margins, form feed, and box. Tiling options (/B) and multiple copies

(/Q) are also here.

Reverse image is not available from PMI.

PMI Other Options ----------------------------------------------------

Options that  are generally set  once and then never  touched are in

column six.  These include the temporary file  name (/T), and  under

"other",  the  default  and  minimum chord  angle  (/Z), plotter D/Y

switch setting (/Y), internal resolution  (/R), pen  overwrite  mode

(opaque lines - /LO), and the pause to load paper option (/X).

Large Paper ----------------------------------------------------------

PrintGL can handle  printing on paper  larger than the usual 8.5x11,

but this takes some extra work. First you need to increase PrintGL's

print window to  cover the  large paper.  This  is  done with the /L

option  or "print window" in PMI.  The print window plus the margins

(/L  or "print margins")  must fit within  the  printer's  printable

area.

And then you must make  sure that your printer recognizes the larger

paper height.  For  continuous  forms paper  this  is  best  done by

setting skip perforation to off,  either by  printer switch or  with

PrintGL prefix codes.  For Canon, Epson, Fujitsu, IBM, and NEC  line

printers (dot  matrix,  ink jet, thermal) use  /J27  79,  or "modify

output" "prefix codes" 27 79.

For cut sheet paper, on Canon,  Epson, Fujitsu,  IBM,  and  NEC line

printers use /J27 67 0 n, where n is the page length in inches.

And for  some  printers you need to increase the right margin past 8

inches. For the Canon BJC this is done with /J27 81 114.

For page  printers, legal size  paper  may  be specified  with these

printer code prefixes: HP-PCL  printers  /J27 38 108 51 65  (/K27 38

108 50 65 to  reset to letter), Canon  LBP  /J27 91  51 50 59 59 112

(/K27 91 51 48  59 59 112 to reset), IBM LaserPrinter /J27 91 70 5 0

3 1 2 1 (/J27 91 70 5 0 3 1 1 1 to reset).

To get  a 10x16  print  window  on  11x17  paper in a  Canon, Epson,

Fujitsu, IBM, or NEC line  printer, use  these options:  /LF10 16 .4

.25 /J27  67 0 17.  You will probably need to adjust the margins (.4

.25) for best centering.

The  maximum  width of the  print  window  on  most narrow  carriage

printers is 8  - 8/(horizontal dpi).  For  300 dpi printers, this is

7.97. At 120 dpi printers it is 7.93. The maximum width of the print

window on most wide carriage printers is 13.6 - 8/(horizontal  dpi).

At 180 dpi this  is 13.55.  At 360 dpi  it is 13.57.  The horizontal

print margin should be set to 0 to get this width.

p-22

Tips On AutoCAD ------------------------------------------------------

There are many ways to use PrintGL with AutoCAD.  Here is a good way

to start.

Configure  AutoCAD for an HP 7550 plotter.  Select output to a file.

Do not do any plotter calibration. Set a specific scale, do not fit.

Do not rotate.  Set  a plotter  window that is larger than  you will

ever  need,  30x30  for  example  (this  will  prevent  AutoCAD from

clipping the  plot).  Set  up your plotter  pen table so  that  each

screen color is assigned a different pen.

With  PrintGL  you can  rotate  (/O  or  "origin"),  magnify  (/M or

"magnification"), assign pens (/C,/S,/W or "pen color", "pen shade",

"pen  width"), and calibrate (/F  or  "modify output" "override  h,v

dpi").  Use /A or "position" "plot center at print window center" to

automatically center your plot in the print window.

Answers --------------------------------------------------------------

If your plot is messed up, with  wraparound lines covering the page,

you have probably exceeded PrintGL's internal coordinate limit. This

happens when the HP-GL  coordinates  exceed 32  inches.  Reduce  the

internal resolution  with "other" "internal  dpi: 762" or /R762  for

coordinates up to 42  inches  or /R508  for  coordinates  up  to  64

inches.

If  you  are having trouble getting  the print  size  or location as

desired, check your /L option or "print window" and "print margins".

The  print  window plus  the print margins must define  an area that

fits in the printable area of the paper.  Printers cannot print over

the entire area of the paper.  They usually require .25 to  .5  inch

margins.

If your prints are coming out not quite to scale:

Check that no calibration is being done by the graphics package.

Use  the "modify output" "h,v size  multiplier:" option or the  /F

option to adjust the print size. See /F - Output Format Option for

information on print size correction.

For the  Canon BJC-800,  the right margin defaults to 8 inches.  Use

/J27 64 27 81 114 to set it to 11.4 inches.

For the  Canon BJ-330 and most wide  carriage printers,  the maximum

print window  width is 13.57  inches.  This is a printer  limitation

that you cannot get around.

If your  printer ejects the paper  before the plot is  finished, you

probably needed to specify a paper length prefix code. For most line

printers use /J27 67 0 n where n is the page length in inches.

p-23

Useful Prefix and Suffix Codes ---------------------------------------

Below,  n  represents any number 0..255, nA represents any number in

ASCII format (1 becomes 49, 2 becomes 50, 43 becomes 52 51, etc.).

Canon, Epson, Fujitsu, IBM, NEC dot matrix and inkjet printers

Reset                    27 64

N line paper             27 67 n

N inch paper             27 67 0 n

17 inch paper            27 67 0 17

Right margin n columns   27 81 n  (not compatible with IBM)

Skip perforation off     27 79

Unidirectional printing  27 85 1

HP-PCL printers

Reset                 27 69

Portrait mode         27 38 108 48 79

Top margin 0          27 38 108 48 69

Skip perforation off  27 38 108 48 76

11 inch paper         27 38 108 50 65

14 inch paper         27 38 108 51 65

11x17 inch paper      27 38 108 54 65

manual feed           27 38 108 50 72

N copies              27 38 108 nA 88  (page printers only)

HP DeskJet 500C

High quality, slow  27 42 111 49 81

No ink depletion    27 42 111 49 68

HP PaintJet XL300

Low quality, fast  27 42 111 45 49 81

PostScript

N copies      nA 32 88 67

Paper tray n  nA 32 88 80

IBM LaserPrinter

11 inch paper, tray 1, n copies  27 91 70 5 0 3 1 1 n

14 inch paper, tray 1, n copies  27 91 70 5 0 3 1 2 n

14 inch paper, manual, n copies  27 91 70 5 0 1 1 2 n

N copies                         27 91 70 5 0 0 0 0 0 n

Canon LBP

Reset                  27 99

11 inch paper          27 91 51 48 59 59 112

14 inch paper          27 91 51 50 59 59 112

Manual feed            27 91 49 113

Automatic feed         27 91 48 113

Full page memory mode  27 59 27 91 50 38 122

N copies               27 91 nA 118

Cursor to top of page  27 91 48 100

Diablo mode            27 58

ISO mode               27 59

HP QuietJet

HP-PCL mode      27 37 65

DIP switch mode  27 37 64

p-24

Color Processing -----------------------------------------------------

Color graphics devices use a variety of color technologies.  PrintGL

supports these basic types.

CMY or cyan/magenta/yellow is used by inkjet printers  with no black

ink. This uses three color planes and give eight pure colors.

CMYK  or cyan/magenta/yellow/black is  used by inkjet and dot matrix

printers.  This  uses four color planes  and give eight pure colors.

Using  black ink instead of a  cyan/magenta/yellow mix gives a  more

neutral black color.

RGB or red/green/blue is used by displays  and  bit maps.  This uses

three color planes and give eight pure colors.

RGBI  or red/green/blue/intensity is used  by displays and bit maps.

This uses four  color  planes.  All  simple colors except black  are

intense.  When  you specify any color mixed  with black at  shade 0,

then instead  of  mixing the colors at 50%, the intensity  is turned

off. This gives fifteen pure colors.

CMYK and RGBI are used by default when they are appropriate. You can

switch to CMY or RGB with the ` suboption of /F.  For displays  this

gives a better picture of how a printer will handle the colors.  For

dot matrix printers it may speed printing by  eliminating the use of

the black ribbon.

/FA!  (300  dpi color  HP-PCL) uses  CMY  color  by  default.  /FA!`

switches to CMYK color to let you  access true  black on the DeskJet

550C.

HP-GL Commands -------------------------------------------------------

PrintGL  supports the 7475 and  7440/17440 command sets  except  for

character set  8 (Katakana), digitize commands, output commands, the

error mask command,  and some device control commands.  In addition,

PG, AF, ES, LO, character set 5, and proportional spaced fonts, from

the 7550,  are supported.  And  forty pens  are  allowed  instead of

eight.

HPGL.DOC  and  HPGL.CHR include more details of HP-GL  and PrintGL's

interpretation. The 7550 manual is the best reference for HP-GL - HP

manual 07550-90001.

p-25

Output Devices -------------------------------------------------------

Canon BJ and BJC Printers, IBM ExecJet

For the BJ130e, BJ5, BJ10, BJ20,  BJ200, BJ300, BJ330, and ExecJet

in Proprinter mode, use /F7.  This is the fastest driver for these

printers. /F7+ gives dark print with line to line overlap.

/FJ works with any Canon BJ printer in BJ130 mode.  The printer is

set to 10 pitch text when finished.

You can  use /FT* with the BJ or ExecJet printers in Epson LQ mode

to get 360 dpi print, but it is very slow.

These printers  have an image  density option (switch  1-9 on  the

BJ130,  1-4  on  the  BJ300,  BJ330,  and  ExecJet).  Low  density

eliminates  every second dot.  To get the full 360 dpi resolution,

use high density.  For the BJ300, BJ330, and ExecJet, also be sure

that the front panel FONT HQ (high quality) light is on.

Use /FB for Canon BJC printers. The prefix code 27 81 114 sets the

right margin to 11.4 inches. /FB+ gives a sparse line overlap.

Canon LBP Printers

/FD  works  with any Canon LBP printer in ISO mode.  At 300 dpi it

takes up to one  meg  of  printer memory to print a full  page  of

graphics. At 150 dpi (/FD-) 512K handles a full page.

It  may help to set the printer memory mode with /J27 59 27 91  50

38 122.

If the printer is in Diablo mode, PrintGL switches to ISO mode. To

switch back to Diablo mode on completion, use /K27 58.

p-26

Display

When you display a plot, the cursor keys scroll the plot, S cycles

between  full,  half  (the  default),  quarter,  and  eighth  page

scrolling, and Esc returns to DOS.

/FC drives a CGA, but does not give color.

The EGA drivers, /FE and /FM, require a 128K or 256K EGA. /FC will

work with a 64K EGA.  /FM is for an EGA  card with a digital (TTL)

monochrome monitor.

/FH supports the Hercules monochrome graphics card.

/FV works with any VGA card.

/FV+ requires a system with 800x600 16  color support.  By default

/FV+ assumes VESA support.  If your VGA card does not support VESA

then add the mode  number for 800x600 16  color mode,  for example

/FV+$5B. Here are mode numbers for common VGA cards ($ means hex).

ATI   $54     Orchid   $6A    Renaissance $64    Trident     $5B

Genoa $29     Paradise $58    Tseng       $29    Video Seven $16

/FV* works with VESA systems that support 1024x768 16 color mode.

For MCGAs, use the VGA driver without color - /FV~.

Display output cannot be redirected to a file  (the  /D option  is

ignored).

Epson, IBM 9 Pin Printers

/FN, /FW, and /F1 cover a wide range of Epson/IBM compatible 9 pin

dot matrix printers.  You need a  cyan/magenta/yellow/black ribbon

to get color prints. The * drivers use 1/240 graphics. /FN+, /FN*,

/FW+,  and  /FW*  use  1/216  indexing  and  do  three  vertically

interleaved passes.  /F1+  and /F1* use  1/144 indexing and do two

vertically interleaved passes. /FW sets line feeds to 1/6 inch.

For  Epson printers, the IBM  Graphics Printer, and other printers

with 1/216 indexing hardware, /FN is preferred. For some not quite

compatible  printers,  /FW works.  For IBM  Proprinters and  other

printers  with 1/144 indexing hardware, /F1 gives better  results.

Some Proprinters do not handle 1/240 graphics correctly.

These drivers use the following escape codes:

esc 3  set n/216 line feeds for all /FW

esc J  index n/216 (n=2 for 1/144 index) for all /FN and /F1

esc L  graphics command for /FN, /FN+, /FW, /FW+, /F1, and /F1+

esc Z  graphics command for /FN*, /FW*, and /F1*

esc r  set ribbon color (only if color is specified)

p-27

Epson, Fujitsu, NEC, Toshiba 24 Pin Printers

/FT  covers  all  Epson LQ printers and  many  compatible printers

(Panasonic in LQ mode and others).  /FF works with  Fujitsu 24 pin

printers in Fujitsu mode.  /F9 works with NEC 24 pin printers. /FO

works  with Toshiba 24  pin printers.  /FO  resets the  line  feed

distance to 1/6 inch on completion.

For all of these  drivers,  you need  a  cyan/magenta/yellow/black

ribbon to get color prints. The + and * drivers use 1/360 graphics

mode, and the *  drivers use 1/360 (1/120  for /FO*)  indexing and

reset the line feed distance  to  1/6 inch on completion.  A clean

paper  path  with equal tension on  each side  is  needed for good

360x360 graphics.

Many 24 pin printers are not capable of 360x360 graphics and a few

cannot handle 360x180 graphics.

The compression mode drivers (!  suboption)  may or  may not be an

improvement with these printers.

The /FT drivers use the following printer escape codes:

esc $    absolute tab

esc * '  graphics command for /FT

esc * (  graphics command for /FT+, /FT*

esc +    set n/360 indexing for /FT*

esc 2    set 1/6 indexing for /FT*

esc J    index n/180 for /FT, /FT+

esc r    set ribbon color (only if color is specified)

HP DeskJet, HP PaintJet XL300, Mannesmann Tally MT92C

/FI works with any black/white DeskJet in portrait mode.

/FA drives the  DeskJet 500C, PaintJet XL300, MT92C, and any color

300 dpi HP-PCL printer that supports data transfer mode 2.

/FA  has three modes and covers different  levels of 300 dpi color

HP-PCL printers.  /FA uses RGB color  and  works with the  DeskJet

500C and Mannesmann  Tally MT92C.  /FA!  uses  CMY color which can

speed printing  in some  cases.  This works with the DeskJet 500C,

DeskJet 550C, and PaintJet XL300.  /FA!` uses  CMYK  color to give

give better blacks on the DeskJet 550C.

p-28

HP LaserJet

/FL works with any LaserJet compatible printer. /FU works with the

LaserJet 2 and any HP-PCL 4 printer. /FI works with the LaserJet 3

and  any  HP-PCL  5 printer.  /FI+ is a 600  dpi  driver  for  the

LaserJet 4.

/FI works with the LaserJet 3 and any HP-PCL 5 compatible printer.

All of these drivers work in portrait mode, which may be  set with

/J 27 38 108 48 79 or you can reset the printer with /J 27 69.

LaserJets with under one meg of memory may not be able to handle a

full page of graphics.  /FU uses less printer  memory than /FI, so

it may be preferred on LaserJet 3s with limited memory.

HP PaintJet

/FP drives 180 dpi PaintJet printers.  The compression mode driver

(/FP!)  uses  data  transfer  mode 2 (instead of 1) and works only

with a PaintJet XL.

/FA! (the DeskJet 500C CMY driver) works with the PaintJet XL300.

HP QuietJet

/FQ works with QuietJets in HP-PCL mode.

HP-GL and HP-GL/2

/F8  outputs  low level  HP-GL, converting  text, arcs,  fill, and

clips to vectors.  The output is HP  7220 and HP-GL/2 pen  plotter

compatible, containing  IN,  IP, SC, SP,  IW, LT, PA,  PU,  and PD

commands.  PG is used for form feeds.  If you specify no form feed

then multiple plots may be appended to the same page.

/F0  drives  HP-GL/2  raster  devices.  Use  /F8  for HP-GL/2  pen

plotters.

You  can specify  the  nominal resolution of  HP-GL or  HP-GL/2 by

appending it to the /F option (for example /F8 300).  When you set

the nominal resolution of HP-GL to other than  1016,  enhanced  IW

command interpretation (HP 7550 enhanced mode) is needed to handle

dashed lines correctly.

For  HP-GL  output  the  width  parameter (/W) is  used  as  a pen

selector, so for each pen in the original HP-GL, you can specify a

new  pen  in  the  output.  The  default  /W  option  for  /F8  is

/W12345678. Shading and color are ignored.

IBM LaserPrinter

/F4 drives any  LaserPrinter in PPDS (native) mode.  /FL will work

if the printer is in HP LaserJet mode.  At 300 dpi  it takes up to

one meg of printer memory to print a full page of graphics. At 150

dpi (/F4-) 512K handles a full page.

p-29

IBM 24 Pin Printers

PrintGL has  three drivers  that  cover  Proprinter X24 compatible

printers.  Use native mode, not AGM. The first driver, /F6, is for

all IBM Proprinter X24s and the 24P.  These printers do 1/144 inch

indexing, and using  them at  180 dpi  results in horizontal white

streaks every  2/3 inch.  The /F6 drivers compensate  for  this by

indexing 19/144 inch per line,  giving  182 dpi.  To work properly

the  printer  must start on a 1/72 boundary.  If you keep the line

feeds at a multiple of 1/72 (1/6, 1/8, 1/9) then this  will always

be the case.

The second X24 compatible driver is /F5 and is  referred to as X24

alternate 1.  This  is for printers that are X24 compatible except

that  they do 1/180 inch indexing.  This is probably the case with

most X24  compatible  printers, including  the IBM Quickwriter and

IBM  2390 and 2391.  /F5* works only  with printers  that do 1/360

indexing, including the 2390 and 2391 but not the Quickwriter.

The third X24 compatible driver  is /FX and is referred  to as X24

alternate 2. This is the same as /F5 except for the vertical units

command.  This driver works with Panasonic 24 pin printers in  X24

mode.

The compression  mode drivers (!  suboption)  may or may not be an

improvement.

IBM Quietwriter 2 and 3

/F2  and /F3 drive the Quietwriter 2 and 3.  Line feeds are set to

1/6 inch on completion.

Kodak Diconix Color 4

/FK drives the Color 4.

Pacific Data Products ProTracer

/FG drives the ProTracer (base model) in IBM Proprinter mode.  The

maximum print window width is 14.97.

p-30

PostScript Output

/FS outputs printable Encapsulated PostScript with no  preview bit

map.  Text, arcs,  fill,  and clips are  converted to vectors,  so

PrintGL is not a general purpose HP-GL to PostScript converter. If

no form feed is specified then "showpage"  is not output, and more

PostScript  may be appended to the page.  PostScript overwrites as

it draws, so overlapping lines appear opaque.

You  can  specify  the  nominal  resolution  of the  PostScript by

appending it to the /F option (for example /FS 300).

You can  specify  the number  of copies  with a  prefix  code that

includes the number of copies followed by ' XC'. For example /J 50

32 88 67 makes 2 (ASCII  050) copies.  You can specify  the  paper

tray with the XP macro.

/FY appends a black/white  TIFF  preview  bit map to a  PostScript

file.  To  use it, create a PostScript file with  PrintGL and then

run PrintGL again, specifying  the same options except /FY instead

of /FS. The destination file is appended by default. /FY assumes a

PostScript file, generated by PrintGL, is being appended.  You may

set the resolution - the default is 72x72 dpi. Here is an example:

PRINTGL sample.plt /Dsample.eps/LB6,6/O2/FS

PRINTGL sample.plt /Dsample.eps/LB6,6/O2/FY

Tektronix ColorQuick

/FR drives the ColorQuick.

p-31

ZSoft PCX Bit Map

/FZ gives ZSoft PCX bit map output.  The format used by PrintGL is

described below.  For  multiple  page plots and tiled  plots use a

three  digit number for the output file (/D option) so  that a new

(sequentially numbered) file will be created for each page.

The file starts with a header composed of 128 byte values:

10,2,1,1,0,0,0,0,

max_x,max_y,                              2 bytes each, in dots

dpi_x,dpi_y,                              2 bytes each

0,0,0,    0,0,192,  0,192,0,  0,192,192,  color palette

192,0,0,192,0,192,192,192,0,192,192,192,

0,0,0,    0,0,254,  0,254,0,  0,254,254,

254,0,0,254,0,254,254,254,0,254,254,254,

0,

color_planes,                             1, 3, or 4

width                                     2 bytes, in bytes

1,0,..,0

The  scan lines follow, in top to bottom order.  For color  plots,

each complete  scan  line  consists  of a blue,  green,  red,  and

intensity scan line.  The intensity line is not included for /FZ`.

1 is the background and 0 the foreground for b/w files. 1,1,1,1 is

the background for color files, with 0 representing the absence of

blue, green,  red, or  intensity.  The bit usage  is reversed with

/LR.  Each  scan line is composed  of these data commands ($ means

hex):

$00..$BF           - 1 byte of bit mapped data ($00..$BF)

$C1 $C0..$FF       - 1 byte of bit mapped data ($C0..$FF)

$C2..$FF $00..$FF  - 2..63 bytes of repeated bit mapped data

The width and height of the bit map are specified by the /L option

plus eight dots. The width is extended to a multiple of sixteen.

p-32

Compatibility and Technical Information ------------------------------

PrintGL and  PMI will run on any  IBM compatible system with  MS-DOS

2.0 or above. It requires up to 256K of free memory. More memory may

speed printing.

By  default, PrintGL uses the BIOS printer interrupt to send data to

the printer. The H1..H4 and X1..X4 drivers send data directly to the

serial port, without using or setting the hardware interrupts.  If a

file or device is specified for output then DOS 2.0 file handling is

used. Messages are written to the screen with DOS CON output.

PMI runs PrintGL via a DOS shell.  It searches the current directory

and then the DOS PATH, calling the first copy of PRINTGL.EXE that it

finds. To save the settings, it makes the same search for PMI.EXE.

PMI can use any text or graphics display  mode that  allows at least

80 columns and 25 lines of text.  In text modes, PMI writes directly

to the display buffer,  and  in  graphics  modes,  BIOS  is  used to

display text. Under DESQview, PMI writes to the DV screen buffer.

PRINTGL.EXE may be compressed with PKLITE, LZEXE, or other EXE  file

compressors.  PMI should  not  be  compressed  because  it  rewrites

PMI.EXE when saving the user settings.

PrintGL  and  PMI were  written  by  Cary  Ravitz and compiled  with

Borland's Turbo Pascal 6.0 and Turbo Assembler 2.0.

User Support ---------------------------------------------------------

Correspondence  about  PrintGL may be sent via  the support bulletin

board, fax, Compuserve, or mail.

BBS:        606-268-0577  1200..9600,N,8,1  24 hours/day

To ask a question, prepare an ASCII text file with the details  of

your question.  Then call the bulletin board and choose the upload

question option.  XMODEM  file transfer  protocol  is required for

this.  Your question will  be assigned  a number (such as  1015) -

remember this  number.  An answer  file (with the assigned number)

will  be posted, usually within  24 hours.  To get an answer, call

the  bulletin board,  choose the download option,  then the answer

option, and then enter the answer number.

Compuserve: Cary Ravitz [70431,32]

Fax:        606-268-0577

If  you hand dial,  the BBS  will answer  and  hang up  (about  20

seconds), then the call is routed  to the fax.  Faxes from outside

the  USA may be answered via  letter.  The  fax may be unavailable

sometimes.

Mail:       Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA