Installation of OPL3-SA Series Device Driver for OS/2 ver3.02

Dec 18th 1996

The set up disk includes the following files.

CARDINFO.DLL	   ：installation information for Audio adapter

OPL3SARC.DLL   ：audio resource information

OPL3SA.HLP　　  ：help file for installation

AUDPLAY.ICO　　：icon of WAVE

MIDIPLAY.ICO　　：icon of MIDI

VIDPLAY.ICO 　  ：icon of Video

CONTROL.SCR 　 ：control file for installation

OPL3SA.SCR　　  ：control file for installation file

GAMEDD.SYS　　 ：device driver of Joystick

OPL3SA.SYS　　   :device driver of OPL3-SA series

GAMEVDD.SYS　　:virtual device driver of Joystick

OPL3VDD.SYS　　 :virtual device driver of OPL3-SA series

OPL3DEMO.EXE　　:Wide Control Application for OPL3-SA3

OPL3DEMO.TXT　　 :Explanation for OPL3DEMO.EXE

1.How to install the driver.

1)install the MMPM/2 (multimedia software support) first to your OS2 because the multimedia application installer is used to install the driver.

2)make sure that there is enough memory in the hard disk to install the driver.(approximately 300byte)

verify that the resource (IRQ,DMA channel, I/O address) the driver will use is not used by other devices.

after booting the OS/2, insert the disk provided by Yamaha.

create the directory so that all files are copied.

Copy all files into the directory the hard disk.

run the multimedia application installer in the multimedia folder by double-clicking.

specify the directory that all files are copied into when the dialog box of is open.

click the “install” button after the driver is selected.

click “yes” when the dialog is open to ask you to modify CONFIG.SYS.

select “1” as the number of driver to be installed then click OK.

select the appropriate options from the option setting dialog according to the system configuration then click OK. Make sure that no resource conflict occurs.

after verifying that the installation has been successful, reboot the system.

- I f you encounter this message:ERROR InitHardware: Can't find a OPL3-SA card !!, this means that OPL3SA is not detected by the system for some reasons. Please make sure there is no resource conflict.

- 2.Options

- When installation has been done successfully, the four line listed below are added to CONFIG.SYS.

- DEVICE=[MMOS2 dir]\OPL3SA.SYS /I:x /D:xy /W:xxx /F:xxx /M:xxx /A:x

- /C:xxx /S:xxx /N:zzz /J:x,yyy

- DEVICE=[MMOS2 dir]\GAMEDD.SYS /J:x,yyy

- DEVICE=[MMOS2 dir]\OPL3VDD.SYS zzz

- DEVICE=[MMOS2 dir]\GAMEVDD.SYS

- Here are explanation for the options.

- (1)/I:x（x:5,7,9,10,11）

- Interrupt for Windows Sound System

- (2)/D:xy（x:0,1,3 y:0,1,3 or select from the combination:10,13,01,03,30,31）

- DMA channel for Windows Sound System

- (3)/W:xxx (xxx:530,604,E80,F40)

- Base I/O address for Windows Sound System

- (4)/F:xxx（xxx:388h,398h,3A8h,3B8h）

- base address for OPL3

- (5)/M:xxx (xxx:0(disable)h,330h,320h,310h,300h)

- Base I/O address for MPU401)

- C:xxx (xxx:F86,390h,620h,910h,A10h)

- Base I/O address for control registers.

- (7)/N:zzz

- name of the device driver

- (8)/J:x,yyy（x:0(Disable）,1(Enable) y:201h,20Ah,20Eh）

- control for Joystick

- (9)/A:x (x:0 (SA2 only), 5,7,9,10)

- Interrupt for MPU401

- (10) /S:xxx (xxx:0(disable), 220,240,260)

Base I/O address for Sound Blaster