W9XNDIS3.DOC

Kingston Technology Company

INSTALLATION GUIDE

Windows 95/98 NDIS 3 Driver

for

EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)

Copyright (C) 1997 Kingston Technology Company.

All rights reserved.

To use QStart under Windows 95/98, you need to run the program in

Windows's MS_DOS mode. To go to MS_DOS mode, click on 'Start', then

click on 'Shut Down'.  Select 'Restart the computer in MS_DOS mode?'.

I. Driver Installation Procedure:

This installation procedure assumes WIN 95/98 is properly installed on

your computer.

Installing Windows 95/98 NDIS 3 Driver

1.

Make sure your computer is turned off. Secure the PCI adapter in

an available PCI slot.

2.

Boot up Windows95/98. The system will detect the presence of the

new PCI Ethernet Hardware and ask you to insert the Manufacturer's

provided diskette. Insert the Kingston Drivers Diskette, change the

directory path to A:\ and continue.

3.

Windows will display the driver name "Kingston EtheRx PCI 10/100

Fast Ethernet Adapter (KNE110TX)". Select the driver and continue.

4.

Follow Windows95/98's on screen instruction to finish the

configuration.

5.

Restart you computer to finish the installation.

After your adapter installation has completed successfully,

you can view and use network resources in Windows 95 by clicking

on Network Neighborhood.  Windows 95/98 will search for and then

display the network resources it finds.  See your Windows 95/98

documentation for more information about using and sharing resources

with Windows 95/98.

III. Setting Up Your Network Components

Your EtheRx PCI adapter is now configured and ready to go.

Now you must verify that you have the following network components

installed on your computer.  You can do this by going into the Control

Panel, and double-clicking on your Network icon.

Client for Microsoft Networks

Client for Netware Networks (if applicable)

IPX/SPX compatible protocol

Kingston EtheRx PCI 10/100 Fast Ethernet Adapter

NETBEUI

File and Printer Sharing for Microsoft Networks

If any of the above components are missing, just click on the "Add"

button, and select the appropriate option as follows:

For additional network client, select Client

For IPX/SPX or additional network protocols, select Protocol

For File and Printer Sharing, select Service

If you made changes to the network components, please restart your

computer so the necessary changes can take affect.

IV. Sharing Resources

You will need to share at least one resource on the network to enable

other computers to see your workstation.  With all folders closed,

double-click on the My Computer icon and place your mouse pointer

over the DRIVE C:, or Hard Disk, icon.  Now click the right mouse

button and select the Sharing option from the pop-up menu.

Note: If you do not have a Sharing option on the menu, please verify

that you have File and Printer Sharing for Microsoft Networks installed

under the Network icon in Control Panel, and restart your computer.

By default, the option will appear as Not Shared;  please change the

setting to Shared As.  At this point, you can assign the access

options and passwords desired.  When finished, restart your computer.

V. Accessing the Network

After restarting your computer, please enter your Windows password.

Your desktop should now have an additional icon called Network

Neighborhood.  By double-clicking on this icon, you should be able

to access the other workstations on the network.

VI. Trouble Shooting

Problem:

Unable to browse the network.

Solution:

Check the following:

1.

Make sure that your device driver does not have a red "X" or yellow "!"

on it in Device Manager.

2.

Check for Link / LED lights on the card / hub (if applicable.)

3.

Try switching cables, or hub ports (if applicable.)

4.

If using Coaxial media, make sure that each network card has a

T-connector, and that you have 50 Ohm terminators on both ends of

the network.

5.

If you are connecting two machines together using RJ45 cabling

only (no hub), make sure your cross-over cable is built correctly,

crossing pins 1&3 and 2&6.  Even though pins 4,5,7, & 8 are not used,

it is mandatory that they be present in the cable. (NOTE: A cross-over

cable is necessary for connecting PC to PC, or hub to hub, unless your

hub has a built-in cross-over switch on one of the rear-panel ports.)

6.

In Windows 95/98, make sure that the Browse Master, in Control Panel,

is not enabled on all workstations.  Automatic settings are recommended

(See File and Printer Sharing for Microsoft Networks for more detail.)

7.

If your network consists of a hub using both BNC and twisted

pair connections, try disconnecting BNC from the network.

8.

Close the Network Neighborhood, and double-click on it after

a few seconds to see if a connection is made.  Sometimes Windows

95/98 takes a minute to establish a network connection.

Problem:

Cannot connect to a Netware Server

Solution:

Please check the Frame Type value in the advanced section of the IPX/SPX

compatible protocol in "Control Panel", "Network."

Problem:

There are no configuration conflicts between the network card and

Windows 95/98 but Windows 95/98 doesn't ask for a network login when it

starts.  When you go into Network Neighborhood you are never able to

see any other system (including yourself) and you always get the

Unable to Browse error message.

Solution:

From Windows 95/98's desktop, click the START button and select RUN.

Type REGEDIT and click on OK.  Windows 95/98 will bring up the

Registry Editor.  Click on EDIT and come down to FIND.  In the box

where it asks you what to search for, type in AUTOLOGON.  Click on

FIND NEXT.  When it finds the folder it will highlight it on the left

side of the screen.  Click on EDIT and come down to RENAME.  You will

be dropped into a box showing the folder's current name.  Put some other

character (any character) in the first position of the file name (just so

that it's no longer called AUTOLOGON.  ~UTOLOGON is fine.)

Press ENTER to save the change, click on REGISTRY, come down to EXIT

then restart the system.  When Windows 95/98 comes back up it should prompt

you for your network login.

Kingston Technology, EtheRx, and QStart are registered

trademarks of Kingston Technology Company.  All others are

trademarks or registered trademarks of their respective holders.