Installation Guide

for

DigiCHANNEL Intelligent

Asynchronous Communications Boards

Universal DOS Device Driver

DOS 2.0 - 5.0

DBI 92000114C

DigiBoard, DigiCHANNEL, PC/Xi, PC/16i, PC/8i, PC/Xe, PC/16e, PC/8e, PC/4e,

PC/2e, COM/Xi, COM/8i and COM/4i are trademarks of DigiBoard.  All other

brand and product names are the trademarks of their respective holders.

c DigiBoard 1992

All Rights Reserved

Information in this document is subject to change without notice and does not

represent a commitment on the part of DigiBoard.

DigiBoard provides this document "as is," without warranty of any kind, either

expressed or implied, including, but not limited to, the particular purpose.

DigiBoard may make improvements and/or changes in this manual or in the product(

s) and/or the program(s) described in this manual at any time.

This product could include technical inaccuracies or typographical errors.

Changes are periodically made to the information herein; these changes may be

incorporated in new editions of the publication.

RESTRICTED RIGHTS:

For non-U. S. Government use:

These programs are supplied under a license.  They may be used, disclosed,

and/or copied only as permitted under such license agreement.  Any copy must

contain the above copyright notice and this restricted rights notice.  Use,

coping , and/or disclosure of the programs is strictly prohibited unless

otherwise provided in the license agreement.

For U.S. Government use:

Use, duplication, or disclosure by the Government is subject to restrictions

as set forth in sub-paragraph (c)(1)(ii) of the Rights in Technical Data and

Computer Software clause of DFARS 52.227-7013.

Table of Contents

Introduction	4

Features	5

Hardware Installation	6

DigiCHANNEL PC/Xe and PC/Xi	7

Memory Start Address	7

I/O Port Address	9

Interrupt Request Line	9

DigiCHANNEL COM/Xi	10

DigiCHANNEL MC/Xi and MCd/Xi	11

Device Driver Installation	12

MC/Xi Board Parameters	13

PC/Xe, PC/Xi and COM/Xi Board Parameters	14

Channel Parameters and Options (All Boards)	16

Setting the Starting Channel Number	16

Driver Support	16

Setting the Channel Parameters	17

Enabling/Disabling the Character Ready Flags	19

Enabling/Disabling the ALT PIN Feature	21

Memory Enable/Disable Option (PC/Xe, PC/Xi Only)	22

Exiting the Set-Up Program	23

Installing the Device Driver	24

Error Messages	25

Introduction

This universal device driver supports the following DigiBoard

DigiCHANNEL asynchronous serial communications boards under

most versions of MS-DOS:

*	PC/Xe	2, 4, 8 and 16 port boards for AT/386 computers

*	PC/Xi	8 and 16 port boards for AT/386 computers

*	COM/Xi	4 and 8 port boards for PC/XT/AT/386 computers

*	MC/Xi	4, 8 and 16 port boards for Micro Channel (PS/2)

computers

*	MCd/Xi	Special DOS version of the MC/Xi

This device driver provides a common programming interface for all of

the DigiCHANNEL boards listed above-with very few restrictions,

software written to support one of these boards will be portable to

systems containing other boards and combinations of boards.

The key to this intercompatibility of boards is the on-board Front-End

Processor Operating System, or FEPOS.  Each of the DigiCHANNEL

intelligent boards has its own FEPOS software that handles the low-

level functions of the board (such as transmitting and receiving individual

characters from and to the on-board data buffers).  The FEPOS

software has been designed so that while the board-level functions are

unique to each board, the system-level interface is common to all of the

boards.

Features

*	Full DOS command-line support (COPY to or from ports, redirect

input or output with "<>", etc.)

*	27 Interrupt 14h BIOS calls

*	6 Interrupt 14h EBIOS calls

*	Receive buffer status (empty or not empty) can be monitored by

checking a flag in host memory

*	Supports all DigiCHANNEL 4, 8 and 16 port intelligent boards

(C/X support will be available in a future release)

*	Supports baud rates of up to 115200 (57600 for the COM/Xi)

Hardware Installation

This section provides the information necessary to configure the

board(s) for use with the universal DOS device driver, and has been

divided into three sub-sections, one for each of the three basic types of

board currently supported by the device driver.  If you are installing a

PC/Xe or PC/Xi board, the configuration information starts on the next

page.  The COM/Xi configuration begins on page 10.  If you are

installing an MC/Xi board, skip to page 11.

DigiCHANNEL PC/Xe and PC/Xi

Install the PC/Xe or PC/Xi board according to the instructions provided

in the Installation Guide and Reference Manual that came with the

board.  The driver supports up to seven PC/Xe and/or PC/Xi boards,

for a maximum of 112 asynchronous serial ports.

Memory Start Address

The universal DOS device driver supports only memory start addresses

which are below the 1 Megabyte boundary, so set the board's DIP

switches for one of the following addresses (see Note 2 on the

following page):

128K Board	(PC/Xi board with jumpers set for 128K)

80000h, C0000h

64K Board	(includes all PC/Xe boards, and PC/Xi boards with

jumpers set for 64K memory)

80000h, 90000h, A0000h, B0000h, C0000h, D0000h or E0000h

8K Board		(see Note 3 on the following page)

80000h

82000h

84000h

86000h

8A000h

8C000h

8E000h

90000h

92000h

94000h

96000h

9A000h

9C000h

9E000h

A0000h

A2000h

A4000h

A6000h

AA000h

AC000h

AE000h

B0000h

B2000h

B4000h

B6000h

BA000h

BC000h

BE000h

C0000h

C2000h

C4000h

C6000h

CA000h

CC000h

CE000h

D0000h

D2000h

D4000h

D6000h

DA000h

DC000h

DE000h

E0000h

E2000h

E4000h

E6000h

EA000h

EC000h

EE000h

Notes:

1.	In most PC configurations, only addresses between C0000h and

DFFFFh are likely to be available.  The recommended starting

address for a 64K board is D0000h.

2.	The memory window size and starting address on newer PC/Xe

boards are set via the DOS device driver configuration program

instead of by DIP switches.

3.	Newer PC/Xe boards (these have one bank of DIP switches in the

endplate instead of two banks on the top edge of the board) can be

configured for an 8K byte or a 64K byte memory window.  The

8K window provides greater flexibility in installation (there are eight

times as many choices for starting addresses) with little or no

degradation in performance.

4.	All PC/Xe and PC/Xi boards in a system may use the same

memory start address.

5.	PC/Xi boards with 256K or 512K bytes of dual ported RAM are

not supported by this device driver, since they cannot be configured

to operate as 128K or 64K boards.

I/O Port Address

Set the board's I/O port address to one of the following:

100, 110, 120, 200, 220, 300 or 320

I/O port addresses must be unique, and may not be shared by any other

device, including other DigiCHANNEL boards.

Interrupt Request Line

Select one of the IRQ numbers listed below, and set the board's DIP

switches accordingly:

3, 4, 5, 7, 10, 11, 12, 15 or None

IRQ numbers must be unique, and may not be shared by any other

device, including other DigiCHANNEL boards.

The IRQ number for newer PC/Xe boards (these have one bank of

DIP switches in the endplate instead of two banks on the top edge of

the board) is set via the DOS device driver configuration program

instead of by DIP switches.

DigiCHANNEL COM/Xi

Install the COM/Xi board according to the instructions  provided in the

Installation Guide and Reference Manual that came with the board.

The driver software supports up to four COM/Xi boards, for a

maximum of 32 asynchronous serial ports.

Set the board's jumpers for one of the following memory start

addresses:

C0000h, C8000h, D0000h or D8000h

COM/Xi memory addresses must be unique, and may not be shared by

any other device, including other DigiCHANNEL boards.

Set the board's I/O port address to one of the following:

100, 110, 120, 200, 220, 300 or 320

I/O port addresses must be unique, and may not be shared by any other

device, including other DigiCHANNEL boards.

Select one of the IRQ numbers listed below, and set the board's

jumpers accordingly:

3, 4, 5, 7, 10, 11, 12, 15 or None

IRQ numbers must be unique, and may not be shared by any other

device, including other DigiCHANNEL boards.

DigiCHANNEL MC/Xi and MCd/Xi

MC/Xi (Standard Version - 128K Dual-Ported RAM)

Install the board according to the instructions in the Installation Guide

and Reference Manual that came with the board.  You will need to

manually set the Memory Start address to C0000h.  Because it takes

the full 128K of system memory addresses from C0000h-DFFFFh,

only one 128K board can be installed, for a maximum of 16

asynchronous serial ports.

MCd/Xi (Special DOS Version - 32K Dual-Ported RAM)

Install the board according to the instructions in the Installation Guide

and Reference Manual that came with the board.  Up to four boards

can be installed, for a maximum of 64 asynchronous serial ports. The

Automatic Configuration program will make address and IRQ

selections that do not conflict with other devices.

MC/Xi (Newer Models - Switchable between 32K and 128K)

Set the memory size jumper for desired dual-ported RAM configuration

(32K is recommended, but not required-see note below).  When 32K

configuration is selected, up to four boards can be installed, for a

maximum of 64 asynchronous serial ports.

NOTE-The standard MC/Xi (128K version) is not recommended

for use with DOS due to potential memory conflicts in the C0000h-

DFFFFh range.

Device Driver Installation

This section describes the configuration and installation of the DOS

device driver.  The procedures described are used for the first-time

configuration of the driver, as well as any future reconfiguration. With

few exceptions, the procedures are identical regardless of which

DigiCHANNEL intelligent board you have installed.  Differences will be

noted as appropriate.

Power up your computer, boot DOS, and load the device driver

software and configuration utility onto your hard disk.  You may place

the files in the root (\) directory, or a subdirectory of your choice.  An

example follows:

C:\> md digi

C:\> cd digi

C:\DIGI> copy a:xidos5.pgm

C:\DIGI> copy a:xidoscfg.exe

Now, while still in the subdirectory containing the files just copied, run

the device driver set-up program:

C:\DIGI> xidoscfg

The set-up program will configure the DOS device driver so that it can

communicate with the boards you have installed, and set the parameters

the boards will use to communicate with remote serial devices.

After configuring the driver with the set-up program for the first time, be

sure to save the configuration.  The set-up program modifies

xidos5.pgm and writes it back to the disk as xidos5.sys.

The following pages describe the use of the set-up program,

xidoscfg.exe.

MC/Xi Board Parameters

If you are installing MC/Xi or MCd/Xi boards, the configuration

program will automatically find the memory and I/O addresses, the IRQ

and number of channels for each board from the PS/2's Programmable

Option Select (POS).  Your initial screen will look something like this:

DigiCHANNEL CONFIGURATION PARAMETERS

Board   Type   Window   Memory   I/O   IRQ   # Board   Start   Driver

#             Size    Window   Port   #    Chnls     Chnl#   Support

----------------------------------------------------------------------

1     MC/Xi    32k    C8000   F1F0    4     8          4     DOS/EBIOS

2     MC/Xi    32k    D0000   F2F0   11     8         12     DOS/EBIOS

----------------------------------------------------------------------

Configuration Selections:                    Use Arrow Keys

Q>uit   C>hannel Parameters   O>ptions

Enter Selection:

To change board parameters, you must boot your system with the

Reference Diskette, and run the IBM configuration program.

To change channel parameters, options, starting channel number and

driver support, skip to page 16.  The following pages deal with PC/Xe,

PC/Xi and COM/Xi board parameters.

PC/Xe, PC/Xi and COM/Xi Board Parameters

If you are installing PC/Xe, PC/Xi or COM/Xi boards, you will see a

screen similar to this:

DigiCHANNEL CONFIGURATION PARAMETERS

Board   Type   Window   Memory   I/O   IRQ   # Board   Start   Driver

#             Size    Window   Port   #    Chnls     Chnl#   Support

----------------------------------------------------------------------

Driver configured for 0 boards.

----------------------------------------------------------------------

Configuration Selections:                    Use Arrow Keys

Q>uit   C>hannel Parameters   O>ptions

Enter Selection:

The first time you run XIDOSCFG, the parameters will all be empty,

because no boards have been configured.

Select A)dd Board.  You will be presented with three options:

A) COM/Xi    B) PC/Xe    C) PC/Xi

Choose the item that corresponds to the first board you wish to install.

(The first board will have the lowest-numbered starting channel.)

Your choice will be entered under "Board Type", and you will be

prompted for a memory window size (except COM/Xi boards, which

are always 32K).  If your board is a newer PC/Xe board (DIP

switches on the end of the board), you can select an 8K or 64K

window.  In most cases, the 8K window is preferable.  If you are

installing an older PC/Xe board (DIP switches on top), you must select

a 64K window.  If you are installing a PC/Xi board, you can select a

64K or 128K window.  Select the window size that matches the

jumper setting of your PC/Xi board (64K is recommended).

Next, you will be asked to select a starting address for the dual ported

memory window.  If your board is a newer PC/Xe board (DIP

switches on the end of the board), the board's address is defined by

this step.  If your board is an older PC/Xe, a PC/Xi or a COM/Xi,

choose the address that corresponds to the setting of the DIP switches

or jumpers on the board.  NOTE-if your window size is 8K, the

starting address selection is in two parts: you will first be asked for a

64K segment (e.g. 80000h, C0000h, D0000h), then for the actual

starting address (e.g. D0000h, D20000h, etc.).  All PC/Xe and PC/Xi

boards may share the same starting address.

When asked for the I/O port address, choose the address that

corresponds to the board's DIP switch or jumper setting.

Next, enter the IRQ number that corresponds to the board's DIP

switch or jumper setting (if your board is a newer PC/Xe, this step

defines the IRQ that will be programmed into the board).

Finally, enter the number of channels on the board.

Repeat the above procedure for any other boards that you wish to

install. If you installed a COM/8i, a PC/2e and a PC/16i, your screen

will now look something like this:

DigiCHANNEL CONFIGURATION PARAMETERS

Board   Type   Window   Memory   I/O   IRQ   # Board   Start   Driver

#             Size    Window   Port   #    Chnls     Chnl#   Support

----------------------------------------------------------------------

1     COM/Xi   32k    C0000   0100    5     8          4     DOS/EBIOS

2     PC/Xe     8k    D0000   0300   15     2         12     DOS/EBIOS

2     PC/Xi    64k    D0000   0320   12     16        14     DOS/EBIOS

----------------------------------------------------------------------

Configuration Selections:                    Use Arrow Keys

Q>uit   C>hannel Parameters   O>ptions

Enter Selection:

By using the arrow keys, you can position the highlighted bar over any

of the parameters for any of the boards.  Each time a parameter is

highlighted, a menu of alternative values is presented, and you are given

the opportunity to select a new value for that parameter.  When a board

number is highlighted, the menu above is displayed, giving you the

opportunity to quit the configuration program, change channel

parameters and options (see the following section), add new boards, or

remove the board whose number is highlighted.

Channel Parameters and Options (All Boards)

Setting the Starting Channel Number

The Starting Channel Number (sometimes referred to as DX-LOW) is

defined as the channel number of the first port on board #1, and is the

lowest channel number that will be supported by the DOS device

driver.  All ports on all boards for which the device driver has been

configured are numbered sequentially, beginning with the Starting

Channel Number.  The configuration display shows the Starting Channel

Number for each board, but it can only be changed for the first board.

To set the Starting Channel Number, use the arrow keys to highlight any

of the Starting Channel Numbers shown on the display.  When

prompted, enter the new number.

Driver Support

The DOS device driver can support DOS calling conventions (using a

COM name such as COM5), Interrupt 14h BIOS calls and EBIOS

functions (Interrupt 14h BIOS and EBIOS functions are described in

INT14.TXT, on the device driver distribution diskette).  Interrupt 14h

support is always included in the driver.  DOS and EBIOS support can

be removed from the driver, if desired, to make the driver more

compact. This is useful for large application programs which use only

the Interrupt 14h routines to communicate with the boards.

Use the arrow keys to highlight the Driver Support item for any board.

You will be given the option of including DOS support, EBIOS

support, both, or neither along with the standard Interrupt 14h support.

Setting the Channel Parameters

Channel parameters are set on a board-by-board basis.  To change the

channel parameters for a board, use the arrow keys to highlight the

number of the board for which you wish to change the parameters, then

press the letter "C" to select C)hannel Parameters.  You will be

presented with a display similar to this:

BOARD 1 PARAMETERS

CHANL    BAUD      MODE       RX FLOW       TX FLOW     NAME

4      9600      8,N,1        NONE        DSR/CTS     COM5

5      9600      8,N,1        NONE        DSR/CTS     COM6

6      9600      8,N,1        NONE        DSR/CTS     COM7

7      9600      8,N,1        NONE        DSR/CTS     COM8

8      9600      8,N,1        NONE        DSR/CTS     COM9

9      9600      8,N,1        NONE        DSR/CTS     COM10

10      9600      8,N,1        NONE        DSR/CTS     COM11

11      9600      8,N,1        NONE        DSR/CTS     COM12

Q)uit    S)et All               Use Arrow Keys

Select Parameter :

Use the arrow keys to highlight the parameter you wish to change.  In

each case you will be presented with the available options for that

parameter.  If you wish to have the same parameters for all channels,

set up one of the channels in the table as desired, then select S)et All

(you need to have the channel number highlighted).  S)et All sets all

parameters of all channels to match the selected channel.

Baud Rate	The following baud rates can be specified with the

configuration utility:  50, 75, 110, 134.5, 150, 200,

300, 600, 1200, 1800, 2400, 4800, 9600, 19200,

38400, 57600, 76800 and 115200 (NOTE-The

maximum baud rate for COM/Xi boards is 57600;

however, they can only reliably read input data at

38400 baud).

Mode		This parameter defines the character length (5, 6, 7 or 8

bits), type of parity checking (odd, even or none) and

the number of stop bits (1, 1.5 or 2) (NOTE-The 1.5

stop bits setting is not available for COM/Xi

boards).

RX Flow		Specifies the type of flow control to use for incoming

data.  Available options are: XON/XOFF (software

handshaking), DTR (Data Terminal Ready-hardware

handshaking) and RTS (Request To Send-hardware

handshaking).  These options may be used singly or in

combination.

TX Flow		Specifies the type of flow control to use for outgoing

data.  Available options are: XON/XOFF (software

handshaking), CTS (Clear To Send-hardware hand-

shaking), DSR (Data Set Ready-hardware handshak-

ing) and DCD (Data Carrier Detect-hardware hand-

shaking).  These options may be used singly or in

combination.

Name		Specifies the DOS name (e.g. "COM5") of each port.

This is the name you would use in DOS commands

such as "copy file.txt com5".  If the device

driver is configured for Interrupt 14h only (see Driver

Support, on page 16), no name is assigned to the port,

and all references must be made by BIOS Interrupt 14h

calls (BIOS Interrupt 14h functions are described in

INT14.TXT, on the device driver distribution

diskette).

Enabling/Disabling the Character Ready Flags

The DOS device driver provides a method of quickly checking the

status of the input buffers (received data) for an empty or not-empty

condition, without having to set up and execute a BIOS Interrupt 14h

function call.  This is done through the Character Ready Flags, which

are bytes in the host computer's RAM (they are located within the body

of the device driver itself), which can be simply checked for a zero or

non-zero condition to determine whether the associated input buffer is

empty or not empty, respectively.

The FEPOS (Front-End Processor Operating System-the on-board

software which controls the actual functions of the board) constantly

monitors the input buffers for which Character Ready Flags are

enabled, and as soon as a buffer changes from empty to not empty, the

board generates an interrupt request (IRQ).  The interrupt handler in the

DOS device driver sets the appropriate Character Ready Flag and

returns control to the application program, which must periodically

check the flags to see if there is data waiting in the input buffer. If a flag

is set (non-zero; the byte will actually contain FFh), there is at least one

character in the receiving buffer.  When the last character has been read

from the buffer, the DOS device driver will automatically reset the flag

to 00h.

Since the location of the flags is dependent upon where the DOS device

driver has been loaded in memory, Interrupt 14h, Function 0Dh - Get

Pointer to Character Ready Flag, has been provided (BIOS Interrupt

14h functions are described in INT14.TXT, on the device driver

distribution diskette).  This function should be run as part of the

initialization of any application program which will use the Character

Ready Flags, and since each channel has its own flag, the function must

be run for each channel that will be monitored via Character Ready

Flags.

The board and driver must be configured for an IRQ line for this feature

to work.

To configure the DOS device driver to use the Character Ready Flags,

use the arrow keys to highlight the number of the board you want to

configure, and select O)ptions from the main menu, then select C)har

Ready Flags.  You will be presented with a screen similar to this:

Board 1 Character Ready Flag Status

Chnl  4 : Disabled        Chnl  5 : Disabled

Chnl  6 : Disabled        Chnl  7 : Disabled

Chnl  8 : Disabled        Chnl  9 : Disabled

Chnl 10 : Disabled        Chnl 11 : Disabled

Change :

T)oggle One Channel  E)nable All Channels  D)isable All Channels  Q)uit

To enable or disable the Character Ready Flag for an individual

channel, select T)oggle One Channel, then enter the number of the

channel to be modified.  If previously disabled, that channel will now be

enabled, and vice versa.  There are also menu items to enable or disable

Character Ready Flags for all channels with a single keystroke. When

all channels have been set as desired, select Q)uit to return to the main

menu.

Enabling/Disabling the ALT PIN Feature

This feature internally swaps the signals DCD (Data Carrier Detect) and

DSR (Data Set Ready).  This is useful for boards that are equipped

with RJ-45 modular jacks for the serial connections.  The DCD signal is

normally on pin 10 of the RJ-45 jack, which makes it inaccessible to 8-

pin RJ-45 plugs.

When the ALT PIN feature is enabled, pin 2 of the 10-pin RJ-45

connector will be interpreted by the driver as the DCD signal. This

corresponds to pin 1 of an 8-pin RJ-45 connector.  Pin 10 then

becomes DSR (and is inaccessible to 8-pin RJ-45 plugs).

To configure the DOS device driver to use the alternate pin assign-

ments, use the arrow keys to highlight the number of the board you

want to configure, and select O)ptions from the main menu, then select

A)lt Pin.  You will be presented with a screen similar to this:

Board 1 Alt Pin Status

Chnl  4 : Disabled        Chnl  5 : Disabled

Chnl  6 : Disabled        Chnl  7 : Disabled

Chnl  8 : Disabled        Chnl  9 : Disabled

Chnl 10 : Disabled        Chnl 11 : Disabled

Change :

T)oggle One Channel  E)nable All Channels  D)isable All Channels  Q)uit

To enable or disable the ALT PIN feature for an individual channel,

select T)oggle One Channel, then enter the number of the channel to

be modified.  If previously disabled for that channel, Alt Pin processing

will now be enabled, and vice versa.  There are also menu items to

enable or disable Alt Pin processing for all channels with a single

keystroke.  When all channels have been set as desired, select Q)uit to

return to the main menu.

Memory Enable/Disable Option (PC/Xe, PC/Xi Only)

The PC/Xe and PC/Xi boards' dual-ported memory is "stackable"-

that is, all PC/Xe and PC/Xi boards in a system can share the same

memory addresses, since only one board's memory is turned on at any

given time.

The DOS device driver normally leaves a board's memory turned on

after writing to that board, and only switches it off when it needs to talk

to a different board with the same memory start address.

Sometimes, however, it is desirable to keep the PC/Xe and PC/Xi

boards' memory turned off when the driver is not actually performing a

read or write operation.  The most common situation where this is

desirable is when an 8-bit board (such as a VGA or EGA card) resides

in the same 128K region (e.g. C0000h-DFFFFh) as the PC/Xe or

PC/Xi.  PC/Xe and PC/Xi boards are 16-bit boards, and when a 16-

bit device and an 8-bit device inhabit the same 128K block, the 8-bit

device can only address even-numbered bytes.  By keeping the PC/Xe

and PC/Xi boards' memory disabled when not in use, this can minimize

potential 8-bit/16-bit memory conflicts.

To configure the DOS driver to leave PC/Xe and/or PC/Xi memory

disabled when not in use, highlight the board you wish to change (use

the arrow keys), select O)ptions, then select L)eave Memory

Enabled.  You will see the following choices:

Current Status : Leave Memory Enabled.

Leave Memory    D)isabled     E)nabled

Enter Selection

After making your selection, you will be returned to the main menu.

Exiting the Set-Up Program

Once all of the parameters have been set as desired for each board, exit

the set-up program by selecting Q)uit from the main menu.  If the driver

is already loaded and the DigiCHANNEL board(s) installed, you will

be asked:

Re-configure the DigiBoard? (y/*) :

If you answer "yes", the changes will be made to the board and driver,

and will take effect immediately.  Next, you will be asked:

Save Changes to Disk for Re-Boot? (y/*) :

Answer "yes" if you wish the changes to become permanent (you must

answer "yes" if this is the first time you have configured the driver).

Now you will be asked:

Quit? (Q/*) :

Select "Q" to return to the DOS prompt.  Any other key will return

you to the main menu of the set-up program.

NOTE-If you reconfigure the DigiCHANNEL board without

saving the changes to disk, the changes will be effective only until

DOS is rebooted.  The driver will then revert to whatever

parameters have been previously set.

Installing the Device Driver

Once the driver has been configured and the file XIDOS5.SYS has

been created, the driver needs to be added to the system configuration.

This is done by adding the line

DEVICE=C:\DIGI\XIDOS5.SYS

to CONFIG.SYS in the root directory (assuming that you have located

the driver file in C:\DIGI).  Use your favorite text editor to do this.

Now reboot your computer.

When the driver is loaded, the following information should be

displayed (depending upon your system configuration and

AUTOEXEC.BAT file, it may scroll by too quickly to read):

DigiCHANNEL Intelligent DOS Driver

Version 4.0.0

Copyright (C) DigiBoard, Inc. 1991

Board 00 - Initializes Successfully.

Board 01 - Initializes Successfully.

(The above is what you would see if you had installed two boards.)

The DOS device driver is now loaded and ready for use.

Error Messages

During the initialization of the DOS device driver and the Digi-

CHANNEL boards (at boot-up time), the following messages might

appear on your console.  In general, unless precisely the same

configuration has been known to work previously, these messages

indicate that the board(s) and driver were not configured with matching

parameters, or that the configuration is in conflict with some other

device in the system.  If the system has been working as configured, and

no other software or hardware changes have been made in the mean

time, these error messages may indicate a board failure.

Board 00 - Port 320h not found.

The driver was configured for a board with an I/O port at 320h, but

could not find a port at that address.

Verify that the board's DIP switches are set for the correct I/O Port

address, as defined with the set-up program.  (If the board is an

MC/Xi, check the POS)

Board 00 - Port at 320 fails hardware reset.

The driver could not reset the board.  This usually indicates a hardware

malfunction, but could also result from an I/O Port address conflict with

some other device in your system.

Reconfigure the DigiCHANNEL board for a different I/O Port address.

Make sure to run the set-up program to reconfigure the driver

accordingly.

Board 00 - Memory size not supported.

This error will generally occur on PC/Xi boards, and indicates that the

boards have more memory than the driver supports.  The DOS device

driver only supports 64K and 128K boards.

Board 00 - Memory Not Found

No memory was found at the address specified during driver

configuration.

Verify that the board's DIP switches are set for the correct Memory

Start address, as defined with the set-up program.  (If the board is an

MC/Xi, check the POS)

Board 00 - Fails BIOS Initialization.

Board 00 - Fails FEPOS Initialization.

Board 00 - Fails Command Buffer Initialization.

These messages indicate that the initialization process failed while the

driver was attempting to load or use on-board code. Failures of this

kind may be caused by a board hardware malfunction or by memory

address conflicts with another device in your system.

Try rebooting your system. If the message still persists, check your

system for possible address conflicts.

Board 00 - Fails IRQ Test - Check IRQ Setting.

This message indicates the board was initalized properly but interrupts

could not be generated by the board. This usually means that the

board's IRQ setting does not match the IRQ you indicated with the set-

up program or that there is a conflict with another device in your

system. Check and rectify any differences or conflicts. If the error

persists, it is probably related to faulty hardware.