CONTROLLING PRINTING FONTS ON THE TANDY DMP130A PRINTER

The DMP130A.ARC file contains a number of small programs

which control and change the different printing fonts on the

Tandy DMP130A printer. To begin with, these programs are

based on the assumption that the printer's DIP switches are

set to the IBM mode.  Counting from switch 1-1 up I set them

as follows:  ON, ON, ON, OFF, OFF, OFF, ON, OFF, ON.

Switches 2-1 and 2-2 are set to OFF.

Your first step is to unarc the DMP130A.ARC file.  You, of

course, wouldn't be reading this if you hadn't already done that.

After you have freed all of the files just insert the disk

into your A drive once you have booted your computer.  Then

type in the command GO.  This command will type the file called

MENU to the screen.  The menu lists a number of the different

printing fonts along with the command that will execute them.

On typing a particular command the printer will accordingly

be set to that font.  When the A> prompt returns type in the

word TEST.  This will send a short printer test paragraph

to your printer which should be printed in the particular

font you selected.  If it is not just type in GO again and

repeat the test.  However, before doing this reach over and

turn your printer OFF and then ON again.

The reason for this is that the printer must always be

in the default (IBM) mode before starting each test.  The very

same code that changes the printer mode from IBM to TANDY also

changes it from TANDY to IBM.  Therefore, if you execute any

of the commands while the printer is in the TANDY mode it will

be reset to the IBM mode.

All of the commands listed on the menu are TANDY mode

fonts.  IBM fonts were not included because they would be just

about the same as the TANDY fonts.  IBM printers, as far as I

know, do not support the italics font.

One of the advantages of controlling the printer with these

COM commands is that it can be done using relatively few bytes.

The DMP130A.ARC file which contains all of the necessary files

plus this document requires only a little over 4000 bytes.

Another advantage is that the files in this grouping can be

transferred to your favorite program disks and used to control

the printer fonts.  An example of this would be if you wanted a

particular program to be able to print in different fonts.  Just

transfer the MENU and other font files to your program disk.  This

will not require a great deal of space.  Then create a GO.BAT or

AUTOEXEC.BAT file which types the MENU file to the screen.  A

GO.BAT would be created as follows:

Type COPY CON GO.BAT  <ENTER>

ECHO OFF  <ENTER>

TYPE MENU   <ENTER>

^Z  <ENTER>

On typing GO the menu will be displayed showing the different

fonts.  All you do is type in the command of your choice.  When the

A> prompt reappears you then load the main program.  Or if you like,

you can create a GO1.BAT file to do the job.

If you wanted to use only one of the particular fonts every

time you used a certain program you could transfer only that par-

ticular font command to the program disk and include the command in

your AUTOEXEC.BAT Or GO.BAT file.  An example of this would be if

you wanted your LOTUS 123 program to print out only in the com-

pressed font.  Copy the STACOMP.COM file to the LOTUS system disk.

Then include this command either in the AUTOEXEC.BAT or GO.BAT

file.  This way you will get compressed printing every time you

use the LOTUS print command.  If you go from there into a program

like SIDEWAYS that utilizes your graphic card you must first turn

your printer OFF and then ON again in order to set it to the IBM

mode.

Bill Newmiller published a fine article in the April, 1988

issue of the 80 MACRO magazine on just how to go about creating

these font command files using the DEBUG program.  I realize that

80 MICRO is no longer in publication.  If you can lay your hands

on this issue turn to page 33.