INSTRUCTIONS FOR DISASM.COM

DISASM.COM  is a two-pass "intellegent"  8080-Z80  disassembler.

It  disassembles   an  Intel  format  hex  file  into  assembler

mnemonics.   It can produce a console  or printer  listing  or a

disk  file  as its  output.   DISASM  is invoked  by typing  the

following:

x:DISASM x:INFILE.TYP [x:OUTFILE.TYP]

where "x:" is a drive name (A: through E:), "INFILE.TYP"  is the

input file, and "OUTFILE.TYP"  is the output  file.  The type of

the input  file must be .HEX,  and may be left off.   If you are

directing  the output of DISASM to a disk file, you only need to

specify  the output file name if it is different  from the input

file name and/or  you are putting  it on a different  drive from

the input file.  The output file type is forced to .ASM, and you

do not have to specify  it.  The drive  name can be left  off of

any file that is on the default system drive.

After you enter the required  command  line, DISASM will ask you

the following questions:

WANT Z80 CODE?

Type  Y if the file  you  are  disassembling  contains  any  Z80

opcodes  that are not part of the 8080 instruction  set.  DISASM

uses "extended  Intel"  mnemonics  when disassembling  Z80 code.

This means that all 8080 instructions  will have 8080 mnemonics,

and  all  Z80  instructions   will  have  8080-like   mnemonics.

Following  these instructions  is a side-by-side  comparison  of

DISASM mnemonics with Zilog mnemonics.

WANT ASCII COMMENTS?

If you answer  Y to this question,  DISASM  will print the ASCII

equivalent of all instructions and data in the comment column of

the output,  preceeded  a semicolon  (;).  This makes it easy to

locate  text data  in a program  you are disassembling.   If you

answer N, the comments are left off.

WANT OUTPUT PUNCHED?

Answer  Y to this question  if you want  to send  the output  of

DISASM  to a disk  file.   The file  will  resemble  an assembly

source  file  and  can  be re-assembled  if  you  wish.   DISASM

examines  the argument  to any  address-type  instruction  (JMP,

CALL, LXI, etc.), and if it lies between the starting and ending

addresses of the program, a label is inserted at the appropriate

place.  Labels are formed by putting  a letter L in front of the

hex address  of the label.   A label  at 123H  would  appear  as

L0123.   This  method   of  labeling   makes  it  easy  to  find

subroutines, etc.

WANT TABS?

This question  is asked  only  if you answer  Y to "WANT  OUTPUT

PUNCHED?".  If you answer Y, DISASM puts tabs between columns in

its output.   If you answer N, it uses spaces  instead  of tabs.

Normally you should use tabs to save disk space.

WANT OUTPUT PRINTED?

This question  is asked  only  if you answer  N to "WANT  OUTPUT

PUNCHED?".   If you answer  Y, the output  of DISASM  is sent to

your LIST device.   If you answer N, it is sent to your console.

In either case, the output  resembles  a .PRN or .LST file, with

the opcodes and data in hex included with the mnemonics.

; THIS IS A SIDE-BY-SIDE COMPARISON OF THE OUTPUT

; OF DISASM AND ZILOG MNEMONICS.

; A SAMPLE OF EACH TYPE OF INSTRUCTION IS INCLUDED.

;

;	DISASM				ZILOG

;

ORG 00100H			ORG 00100H

; 8-BIT LOAD GROUP

L0100:				L0100:

MOV	A,B			LD	A,B

MOV	A,M			LD	A,(HL)

MOV	A,[X+001H]		LD	A,(IX+001H)

MOV	M,A			LD	(HL),A

MOV	[X+002H],A		LD	(IX+002H),A

MVI	A,003H			LD	A,003H

MVI	M,004H			LD	(HL),004H

MVI	[X+005H],006H		LD	(IX+005H),006H

LDA	L0100			LD	A,(L0100)

STA	L0100			LD	(L0100),A

LDAX	B			LD	A,(BC)

STAX	D			LD	(DE),A

LDAI				LD	A,I

LDAR				LD	A,R

STAI				LD	I,A

STAR				LD	R,A

; 16-BIT LOAD GROUP

L0121:				L0121:

LXI	B,00001H		LD	BC,00001H

LXI	X,00002H		LD	IX,00002H

LBCD	L0121			LD	BC,(L0121)

LDED	L0161			LD	DE,(L0161)

LHLD	L0179			LD	HL,(L0179)

LIXD	00003H			LD	IX,(00003H)

LIYD	00004H			LD	IY,(00004H)

LSPD	00005H			LD	SP,(00005H)

SBCD	00006H			LD	(00006H),BC

SDED	00007H			LD	(00007H),DE

SHLD	00008H			LD	(00008H),HL

SIXD	00009H			LD	(00009H),IX

SIYD	0000AH			LD	(0000AH),IY

SSPD	0000BH			LD	(0000BH),SP

SPHL				LD	SP,HL

SPIX				LD	SP,IX

SPIY				LD	SP,IY

PUSH	D			PUSH	DE

PUSH	Y			PUSH	IY

POP	H			POP	HL

POP	X			POP	IX

; EXCHANGE, BLOCK TRANSFER AND SEARCH GROUP

L0161:				L0161:

XCHG				EX	DE,HL

EXAF				EX	AF,AF'

EXX				EXX

XTHL				EX	(SP),HL

XTIX				EX	(SP),IX

XTIY				EX	(SP),IY

LDI				LDI

LDIR				LDIR

LDD				LDD

LDDR				LDDR

CCI				CPI

CCIR				CPIR

CCD				CPD

CCDR				CPDR

; 8-BIT ARITHMETIC AND LOGICAL GROUP

L0179:				L0179:

ADD	A			ADD	A,A

ADD	M			ADD	A,(HL)

ADD	[X+001H]		ADD	A,(IX+001H)

ADI	002H			ADD	A,002H

ADC	B			ADC	A,B

ACI	003H			ADC	A,003H

SUB	C			SUB	C

SUI	004H			SUB	004H

SBB	D			SBC	A,D

SBI	005H			SBC	A,005H

ANA	E			AND	E

ANI	006H			AND	006H

ORA	H			OR	H

ORI	007H			OR	007H

XRA	L			XOR	L

XRI	008H			XOR	008H

CMP	M			CP	(HL)

CPI	009H			CP	009H

INR	A			INC	A

INR	M			INC	(HL)

INR	[Y+00AH]		INC	(IY+00AH)

DCR	B			DEC	B

DCR	M			DEC	(HL)

DCR	[X+00BH]		DEC	(IX+00BH)

; GENERAL PURPOSE ARITHMETIC AND CONTROL GROUP

DAA				DAA

CMA				CPL

NEG				NEG

CMC				CCF

STC				SCF

NOP				NOP

HLT				HALT

DI				DI

EI				EI

IM0				IM	0

IM1				IM	1

IM2				IM	2

; 16-BIT ARITHMETIC GROUP

L019F:				L019F:

DAD	B			ADD	HL,BC

DADC	D			ADC	HL,DE

DSBB	H			SBC	HL,HL

DADX	B			ADD	IX,BE

DADY	D			ADD	IY,DE

INX	H			INC	HL

INX	X			INC	IX

DCX	D			DEC	DE

DCX	Y			DEC	IY

; ROTATE AND SHIFT GROUP

L01AE:				L01AE:

RLC				RLCA

RAL				RLA

RRC				RRCA

RAR				RRA

RLCR	A			RLC	A

RLCR	M			RLC	(HL)

RLCR	[X+001H]		RLC	(IX+001H)

RALR	B			RL	B

RRCR	C			RRC	C

RARR	D			RR	D

SLAR	E			SLA	E

SRAR	H			SRA	H

SRLR	[Y+002H]		SRL	(IY+002H)

RLD				RLD

RRD				RRD

; BIT SET, RESET, AND TEST GROUP

L01CC:				L01CC:

BIT	1,A			BIT	1,A

BIT	2,M			BIT	2,(HL)

BIT	3,[X+004H]		BIT	3,(IX+004H)

SET	5,A			SET	5,A

SET	6,M			SET	6,M

SET	7,[Y+008H]		SET	7,(IY+008H)

RES	1,A			RES	1,A

; JUMP GROUP

L01DE:				L01DE:

JMP	L019F			JP	L019F

JZ	L01AE			JP	Z,L01AE

JNZ	L01CC			JP	NZ,L01CC

JC	L019F			JP	C,L019F

JNC	L01AE			JP	NC,L01AE

JPO	L01CC			JP	PO,L01CC

JPE	L019F			JP	PE,L019F

JP	L01AE			JP	P,L01AE

JM	L01CC			JP	M,L01CC

JMPR	L01DE			JR	L01DE-$

L01FB:				L01FB:

JRZ	L01DE			JR	Z,L01DE-$

JRNZ	L01FB			JR	NZ,L01FB-$

JRC	L020A			JR	C,L020A-$

JRNC	L020A			JR	NC,L020A-$

DJNZ	L01DE			DJNZ	L01DE-$

PCHL				JP	(HL)

PCIX				JP	(IX)

PCIY				JP	(IY)

; CALL AND RETURN GROUP

L020A:				L020A:

CALL	L01DE			CALL	L01DE

CZ	L01FB			CALL	Z,L01FB

CNZ	L020A			CALL	NZ,L020A

CC	L0121			CALL	C,L0121

CNC	L0161			CALL	NC,L0161

CPO	L0179			CALL	PO,L0179

CPE	L019F			CALL	PE,L019F

CP	L01AE			CALL	P,L01AE

CM	L01CC			CALL	M,L01CC

RET				RET

RZ				RET	Z

RNZ				RET	NZ

RC				RET	C

RNC				RET	NC

RPO				RET	PO

RPE				RET	PE

RP				RET	P

RM				RET	M

RETI				RETI

RETN				RETN

RST	2			RST	10H

; INPUT AND OUTPUT GROUP

IN	001H			IN	A,001H

INP	A			IN	A,(C)

INI				INI

INIR				INIR

IND				IND

INDR				INDR

OUT	002H			OUT	002H,A

OUTP	B			OUT	(C),B

OUTI				OUTI

OTIR				OTIR

OUTD				OUTD

OTDR				OTDR

END				END

#2H			OUT	002H,A

OUTP	B			OUT	(C),B

OUTI				OUTI

OTIR				OTIR

OU