Rancho Technology, Inc.

10783 Bell Court

Rancho Cucamonga, CA  91730

Dear Valued Customer,

FMTFLOPT is a Floptical diskette formatter that will allow you to format

21Mb, 1.44Mb and 720Kb floppy diskettes in a Floptical drive. If you have a

Floptical drive in the system, and you will be formatting 21Mb diskettes, you

must load FMTFLOPT.EXE first.

To load FMTFLOPT, type the following at your DOS PROMPT:

FMTFLOPT <Enter>

This will set the default parameters and will enable you to use DOS'

FORMAT.COM program to format Floptical 21Mb diskettes.  To format a 21Mb

diskette, place the diskette into the drive, then enter the following at your

DOS PROMPT:

FORMAT d: <Enter>

where d: is the drive letter for the Floptical drive.  Then follow the

instructions on your screen and your diskette will soon be formatted. To

format more 21Mb diskettes at a later time, simply issue the FORMAT command.

FOR ADVANCED USERS

------------------

The command-line specification that is used to load FMTFLOPT from the DOS

prompt or from the AUTOEXEC.BAT file is as follows:

FMTFLOPT [/?] [/D[+|-]] [/L[+|-]] [/T[+|-]] [/I[<interleave>]]

/?  Displays a help screen similar to this one

/D  Reports/Changes the format type of the disk (bootable/not bootable)

/L  Reports/Changes the forced low level format flag (on/off)

/T  Reports/Changes the translation status of FMTFLOPT (on/off)

/I  Reports/Changes the interleave for low level format

<interleave> is the new interleave value to set for low level format [0..7]

The four optional parameters are initialized to their default settings upon

initial installation of FMTFLOPT as follows:

Formatted disks are Bootable

Low Level Format status is OFF

Translation status is ON

Verify status is OFF

Current interleave is: 1

These parameters may be changed at any time. The status of each may be viewed

any time following initial installation by type FMTFLOPT <Enter>. Below is a

detailed description of the parameters.

1.  Formatted Disks

The /D parameter will allow you to make your 21Mb diskettes bootable or

non-bootable.

/D+ This option will create a 12-bit File Allocation Table (FAT), making your

21Mb diskette bootable. Once you set this option, however, you MUST use

the /s parameter on the DOS FORMAT command line to make a system diskette.

/D- This option will create a 16-bit FAT, making your VHD diskette

non-bootable, or data-only. A data-only diskette will have more storage

space for programs, data and other files. Once you set this option, DO

NOT use the /s parameter on the DOS FORMAT command line to make a system

diskette.

2.  Low-Level Format

FMTFLOPT will automatically low-level format a 21Mb diskette that is not

formatted. Nevertheless, the /L parameter will allow you to force a low-

level format of your 21Mb diskette when required.

/L+ This option will set the low-level format flag to be on. Before starting

a low-level format, FMTFLOPT will prompt you to confirm your intention of

low-level formatting, which will take approximately 20 minutes. When you

set this option and use the DOS 3.x/4.x FORMAT command, your 21Mb

diskette will be low-level formatted when you FORMAT (drive): <Enter>,

whether or not it has previously been low-level formatted. When you set

this option and use the DOS 5.x FORMAT command, you need to type

FORMAT (drive letter): /U <Enter> to low-level format your 21Mb diskette.

Otherwise, it will be high-level formatted.

WARNING: The low-level format prompt is not displayed under Windows 3.x

or Windows 3.x DOS prompt. Once you low-level format your 21Mb diskette,

there is no way to recover the data on it!

/L- This option will set the low-level format flag to be off.

3.  Translation

The /T parameter will allow you to turn on or turn off the terminate-stay-

resident (TSR) FMTFLOPT program.

/T+ This option will turn the FMTFLOPT program on.

/T- This option will turn FMTFLOPT program off. If you experience any

problems with your applications, type FMTFLOPT /T- <Enter> to see if the

problem goes away.

4.  Verify

The /V parameter will allow you to speed-up or slow-down the formatting

process.

/V+ This option will allow you to slow-down the formatting procedure; that

is, display the percentage completed prompt incrementing at a speed that

can be seen by the the human eye.

/V- This option will allow you to speed-up the formatting procedure; that is,

display the percentage completed prompt incrementing very quickly.

5.  Interleave

The /I[<interleave>] parameter will allow you to set the interleave for

low-level format.

/I0 This option will format with the default interleave of the drive.

/I1 This option will format with a 1:1 ratio--the fastest interleave at which

a drive can operate.

NOTE: Interleaves greater than 7 are not allowed.

Sample Command Lines:

To format a 21Mb diskette with optional parameters at default settings (i.e.

/D+ /L- /T+ /V- /I1), type at the DOS prompt:

FMTFLOPT <Enter>

FORMAT (drive letter): <Enter>

To format a non-bootable 21Mb diskette with the other parameters at default

settings, type at the DOS prompt:

FMTFLOPT /D- <Enter>

FORMAT (drive letter): <Enter>

To low-level format a non-bootable 21Mb diskette with default interleave

under DOS 3.x/4.x, type at the DOS prompt:

FMTFLOPT /D- /L+ /I0 <Enter>

FORMAT (drive letter): <Enter>

To low-level format a bootable 21Mb diskette with an interleave of 1 under

DOS 5.x, type at the DOS prompt:

FMTFLOPT /D+ /L+ /I1 <Enter>

FORMAT (drive letter): /U /S <Enter>

To low-level format a non-bootable 21Mb diskette with an interleave of 2

under DOS 5.x, type at the DOS prompt:

FMTFLOPT /D- /L+ /I2 <Enter>

FORMAT (drive letter): /U <Enter>