%OP%DETime and date management

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,12,80%%C%%H1%%H2%T i m e   a n d   d a t e   m a n a g e m e n t%H2%%H1%

The applications within the Z88 frequently need to manipulate times and dates.

The Clock, Calendar and Diary are the obvious exmamples; also the Filer stores

creation and "last updated" dates in its filing system. Hence a fairly

comprehensive set of routines are provided to handle them. Before discussing

the routines themselves, it is worth explaining how dates and times may be

represented.

Within the machine, dates are represented by 3-byte unsigned integers which

represent the number of days since the conventional day zero, which is Monday

23rd November 4713 BC! This number of days figure is consistent from the point

of view of the New Style (Gregorian) calendar, so will not tally with

historical dates before 14th September 1752 (Britain) or 14th October 1582

(continental Europe) unless the dates have been retrospectively corrected (like

George Washington's birth date). However, it should be entirely correct from

its chosen point of view, incorporating the following rules to deal with leap

years:

The accuracy range is 23/11/4713 BC to 31/12/18253 AD.

Routines provided to convert between this rather inconvenient format and

either:

1) A zoned integer representing a human date conveniently, split over registers

as follows:

2) An ASCII string, with various options (leading blanks, American format,

century [ie. 88 or 1988], month in full [ie. Dec or December] etc).

Times are internally represented as unsigned 3-byte integers representing the

number of 10ms (1/100 of a second) intervals since the start of the day.

Routines exist to convert between this format and an ASCII string, again with

various options. Further routines are provided to read or set the machine time

and date. The available routines are as follows, their specifications can be

found in "System Calls Reference":

GN_GDT

GN_PDT

GN_DIE

GN_DEI

GN_GMD

GN_PMD

GN_GTM

GN_PTM

GN_GMT

GN_PMT

GN_MSC

GN_SDO

%H2%Example%H2%

_______________________________________________________________________________

include ":*//time.def"

include ":*//stdio.def"

; simple example to display the current date

; assumes IY points to at least 30 bytes of available memory

; this memory should not lie in segment 2

_______________________________________________________________________________

%P0%

%CO:B,12,68%

Every year is a normal year (365 days) except

every 4th year is a leap year (366 days) except

every 100th year is a normal year except

every 400th year is a leap year except

every 3200th year is a leap year except

every 80000th year is a normal year.

C (bits 7 to 5)

C (bits 4 to 0)

B

DE

convert an ASCII string to an internal date

convert an internal date to an ASCII string

convert from internal to external format

convert from external to internal format

fetch current machine date

set current machine date

convert an ASCII string to an internal time

convert an internal time to an ASCII string

fetch current machine time

set current machine time

convert real time to elapsed time

output date and time to standard output

push iy

pop  de

call_oz(GN_GMD)

push iy

pop  hl

ld   a, 240

ld   b, 15

ld   c, '.'

call_oz(GN_PDT)

xor  a

ld   (de),a

push iy

pop  hl

inc  hl

inc  hl

inc  hl

call_oz(GN_SOP)

call_oz(GN_NLN)

ret

%CO:C,12,56%%CO:D,12,44%

the day of the week (1=Monday, 7=Sunday)

the day of the month (1 to 31, obviously)

the month (1=january, 12=december)

a signed year number relative to 0 AD

; time & date call definitions

; standard I/O call definitions

; buffer for date

; get machine date

; DE = DE(in) + 3

; source date

; century output, C = interfield delimiter

; expanded day and month (no AD/BC)

; interfield delimiter

; put date in ASCII format

; null-terminate ASCII string

; adjust string buffer address

; of converted string

; write string to std. output

; new line

%CO:E,12,32%%CO:F,12,20%%CO:G,12,8%